BUILD 5

DESIGN OVERVIEW

Yes. This is still the right Build 5 concept, and now that Build 3 is frozen, we can make Build 5 much more precise.

The key design rule should be:

Quick, Full, and Learn are not three separate knowledge bases. They are three views of the same activity object.

That prevents duplication and keeps the interface fast.

Build 5 structure I would lock

Quick Mode = field execution

This is the default view.

It should show only the things you are most likely to need while standing beside the work:

Before

- immediate prerequisites

- gate status

- anything about to be concealed

Inspect

- compact quality checkpoint list

Evidence

- minimum records/photos/measurements worth capturing

Interfaces

- only the most relevant direct cross-links

Watch For

- typical visual risks

Don't Miss

- 1–3 critical reminders

Status guidance

- Accept / Review / Issue

- but as guidance, not live project tracking

That should fit comfortably on one tablet screen with a bit of scrolling.

Full Mode = complete inspection workflow

This is where the Build 2 structure appears almost intact:

Requirements
Planning
Document Control
Material / Equipment Control
Inspection / Testing
Records / Evidence
NC / Deficiency
Corrective Action
Verification
Closure
Reporting / Analysis
Quality Checkpoint
Communications
Outputs

And the Build 3 navigation remains available:

Before | Gates | Interfaces | Testing | Commissioning | After | Closeout

This is the page you use before a major inspection, when unfamiliar with the work, or when something has gone wrong.

Learn Mode = understanding

This should not just reproduce Full Mode with more words.

It should answer:

- What is this activity?

- Why does it matter?

- What are the key terms?

- What commonly goes wrong?

- Why are those failures significant?

- What does a good inspection look like?

- What are practical examples?

- What interfaces usually cause trouble?

- What specialist boundaries matter?

That makes Learn genuinely educational rather than a duplicate checklist.

Activity Mode

I still strongly recommend this.

The important distinction is:

Browse by system tells you where the work belongs.
Activity Mode tells you what is happening right now.

So the homepage can offer workflows such as:

- Concrete Pour

- Wall Closure

- Ceiling Closure

- Roof Inspection

- Roof Penetration

- Underground Service Before Backfill

- Equipment Installation

- Equipment Start-Up

- Pressure Test

- Firestop Inspection

- Deficiency Walk

- Final Acceptance Review

These should be composite workflows assembled from Build 3 links, not separate duplicated content.

For example:

Concrete Pour
→ Formwork
→ Reinforcement
→ Embedded Items
→ MEP cast-ins
→ Pre-Pour Gate
→ Concrete testing
→ Curing/post-pour

Pre-Concealment Mode

This should be a first-class shortcut, not hidden inside Activity Mode.

I would call it:

BEFORE CLOSING / COVERING

Then let the user choose:

- Concrete Pour

- Wall

- Ceiling

- Roof Layer

- Underground / Backfill

- Fire-Rated Assembly

- MEP Concealment

Each screen is built from the applicable gate object.

So for Wall:

- framing

- backing

- insulation

- mechanical rough-in

- electrical rough-in

- penetrations

- firestop

- dampers where applicable

- access

- required tests

- photos/evidence

- unresolved deficiencies

That comes directly from G-INT-01 and G-MEP-01, not from another manually written checklist.

Watch For

This should be highly visual and risk-oriented.

Not ten generic items.

Usually around 3–6.

Example, Firestopping:

- oversized opening

- wrong system for penetrating item

- incorrect annular space

- missing backing

- insufficient installed depth

- later trade damage

Example, Roof Membrane:

- wet substrate

- puncture

- poor lap/seam

- incomplete termination

- damaged flashing

- traffic damage after acceptance

This is where your eye gets trained.

Don't Miss

Even shorter.

One to three items maximum.

Examples:

Concrete

Verify every embed and conduit before the pour.

Air Barrier

Check transitions and penetrations, not just the flat membrane.

Electrical

Physical QC does not replace authorized electrical acceptance.

Firestop

Confirm the installed condition matches an approved firestop system before concealment.

This should be visually prominent.

QC Think

Keep it.

I would make the five questions fixed across the whole app:

- What governs this work?

- What will become hidden or difficult to access next?

- What other system or trade can this affect?

- What evidence proves acceptable execution?

- What happens next if I accept this condition?

And after Step 6, I would add one more:

- Has anything changed since this was last inspected or tested?

That catches invalidation.

One change I would make

I would not put Related Works, Interfaces, Before/After, Testing, and Closeout all as permanent large blocks inside Quick Mode.

That would make Quick Mode crowded.

Instead, Quick should show:

Related: 4

and tapping it expands:

- Before

- Interfaces

- Testing

- After

Same data, less visual noise.

Recommended Quick Mode layout

Something like:

FIRESTOPPING
Quick | Full | Learn

Before Proceeding
☐ Rated assembly identified
☐ Approved firestop system
☐ Penetration/opening acceptable
☐ Correct materials

Inspect
☐ Penetrating item
☐ Opening / annular space
☐ Backing/fill
☐ Required depth/configuration
☐ Complete seal
☐ Identification

Evidence
☐ Location
☐ System reference
☐ Photo before concealment

⚠ Watch For
Oversized opening · wrong system · missing backing · insufficient depth

❗ Don't Miss
Match the actual penetration condition to an approved system.

Related (5)
QC Think

That is field-usable.

Full Mode layout

Full Mode should use collapsible sections rather than one endless page:

Requirements ▾
Planning ▾
Document Control ▾
Material Control ▾
Inspection / Testing ▾
Evidence ▾
NC / Deficiency ▾
Corrective Action ▾
Verification ▾
Closure ▾
Reporting ▾
Communications ▾
Outputs ▾

Only one or two need to be open at once.

Learn Mode layout

Learn could be:

What it is
Why it matters
Key terminology
How good QC looks
Common failures
Practical example
Interfaces to understand
Specialist boundary

That is enough.

Build 5 should now be broken into these exact steps

I would do:

5A — View definitions
Freeze exactly what Quick, Full, Learn show.

5B — Quick-mode extraction rules
Define how Build 2 content becomes concise field checks.

5C — Full-mode presentation
Define collapsible section behavior.

5D — Learn-mode presentation
Define educational structure.

5E — Activity Mode
Define composite workflows.

5F — Pre-Concealment Mode
Map gates into shortcut workflows.

5G — Watch For / Don't Miss / QC Think
Freeze rules and limits.

5H — One complete prototype activity
Use something complex like 10.3 Firestopping or 8.9 Mechanical Penetrations to prove the model.

Then once that works, we can apply it across the whole framework without redesigning anything.

So yes: this is the right Build 5. The next move should be 5A — formally define Quick / Full / Learn and freeze exactly which fields belong in each view.

Yes. 5A should be treated as a presentation contract. We are deciding exactly what information belongs in Quick, Full, and Learn, what is shared between them, and—equally important—what must not appear in each.

BUILD 5 — STEP 5A

QUICK / FULL / LEARN VIEW DEFINITIONS

The fundamental rule is:

One activity. One underlying knowledge object. Three depths of presentation.

So:

QUICK = Help me inspect this now.
FULL = Help me control this work completely.
LEARN = Help me understand this work and the quality reasoning behind it.

They are not three independent pages.

1. UNIVERSAL ACTIVITY HEADER

The top of the page should remain essentially identical regardless of mode.

Example:

10.3 — FIRESTOPPING

QUICK | FULL | LEARN

Purpose: Maintain the required fire-resistance performance at penetrations through rated assemblies.

Then compact controls:

Related Works · Before/After · Gate · Interfaces

These should be accessible from every mode because navigation is independent of depth.

So the user can switch:

Firestop → Electrical Penetration → Wall → Fire-Rated Assembly → Wall Closure

without caring which mode they are currently using.

2. QUICK MODE — PURPOSE

Quick answers:

I'm standing beside the work. What do I need to verify right now?

It should therefore contain only field-actionable information.

No long explanations.

No management analysis.

No detailed requirements discussion.

No paragraphs unless absolutely necessary.

3. QUICK MODE — EXACT CONTENT

I recommend exactly seven primary blocks.

① BEFORE

Question:

What must I confirm before inspecting/accepting this work?

Sources:

- critical prerequisites;

- applicable gate;

- material approval;

- required documentation;

- pre-concealment requirements.

Example:

BEFORE

☐ Approved firestop system available
☐ Rated assembly identified
☐ Penetrating item identified
☐ Opening/configuration acceptable
☐ Correct approved materials available

This should generally be around 3–7 checks.

4. QUICK — INSPECT

This is the heart of Quick Mode.

Source primarily:

Build 2 Quality Checkpoint + critical Inspection/Test checks

Example:

INSPECT

☐ Correct penetrating item
☐ Opening dimensions/configuration
☐ Annular space
☐ Backing/fill material
☐ Required depth/thickness
☐ Complete seal
☐ Surface condition
☐ Identification/label

Generally:

5–10 checks

If we routinely reach 15–20, Quick Mode has failed.

The detailed checks belong in Full.

5. QUICK — EVIDENCE

Question:

What evidence should I leave with?

Only the essential field evidence.

Example:

EVIDENCE

☐ Exact location / ID
☐ Approved system reference
☐ Photo before concealment
☐ Completed inspection record where required

Usually 2–5 items.

This is deliberately different from Full Mode's complete Records/Evidence section.

6. QUICK — WATCH FOR

⚠ WATCH FOR

Usually 3–6 common visual problems.

Example:

- oversized opening;

- wrong system;

- excessive annular space;

- missing backing;

- insufficient fill depth;

- damaged completed firestop.

These aren't checkboxes.

They train visual attention.

7. QUICK — DON'T MISS

❗ DON'T MISS

Maximum:

1–3 items.

Ideally 1–2.

Example:

Match the actual penetration condition to an approved system.

Capture evidence before concealment.

If everything becomes "Don't Miss," nothing is Don't Miss.

8. QUICK — NEXT / GATE

Quick should tell you whether something important happens immediately afterward.

Example:

NEXT

Wall Closure

⚠ Firestop is part of the pre-concealment condition.

Or:

NEXT

Concrete Placement

⚠ Complete applicable pre-pour release before proceeding.

This answers one of the most important field questions:

What happens if I accept this?

9. QUICK — RELATED

Keep this collapsed by default.

RELATED (6) ›

Opening it might show:

Before

- Fire-Rated Wall

- MEP Rough-In

Interfaces

- Mechanical Penetrations

- Electrical Penetrations

- Universal Penetration Workflow

After

- Wall Closure

That data comes from Build 3.

We don't rewrite it.

10. QUICK — QC THINK

Permanent small action:

QC THINK

Opens the six universal questions:

- What governs this?

- What could become hidden?

- What other work does this affect?

- What evidence proves compliance?

- What happens next if I accept it?

- Has anything changed since it was last inspected/tested?

QC Think should be available from every Quick page.

11. QUICK MODE — FROZEN STRUCTURE

Therefore:

ACTIVITY

QUICK | FULL | LEARN

BEFORE

INSPECT

EVIDENCE

⚠ WATCH FOR

❗ DON'T MISS

NEXT / GATE

RELATED (collapsed)

QC THINK

That's it.

I would resist adding more.

12. WHAT QUICK MODE MUST NOT CONTAIN

Explicit exclusion is important.

Quick should not normally show:

- detailed planning;

- complete document-control procedure;

- full material receiving procedure;

- detailed NC lifecycle;

- complete corrective-action workflow;

- reporting/analysis;

- lengthy explanations;

- terminology definitions;

- long practical examples;

- detailed communications;

- complete outputs;

- every possible interface;

- every possible deficiency;

- every conditional specialist consideration.

Those exist elsewhere.

This is how we keep Quick genuinely quick.

13. FULL MODE — PURPOSE

Full answers:

How do I control this activity thoroughly from preparation through closure?

This is essentially your professional QC operating framework.

Use cases:

- preparing tomorrow's inspection;

- unfamiliar work;

- major/high-risk work;

- resolving an issue;

- reviewing whether anything was missed;

- preparing for a hold/witness point;

- complex coordination;

- learning CRT/project requirements and mapping them against your process.

14. FULL MODE — EXACT CONTENT

Full should preserve our complete canonical activity schema.

I recommend the following order.

① QUALITY OBJECTIVE

Short.

What are we trying to achieve?

② REQUIREMENTS

Where do I get the acceptance criteria?

- drawings;

- specifications;

- approved shop drawings;

- PIE/PRIE;

- procedures;

- manufacturer requirements;

- approved technical changes;

- applicable specialist requirements.

③ PLANNING

How do I prepare?

- scope;

- sequence;

- timing;

- interfaces;

- hold/witness requirements;

- access;

- testing;

- evidence planning.

④ DOCUMENT CONTROL

Am I working from the correct information?

⑤ MATERIAL / EQUIPMENT CONTROL

Is what is being installed actually what was approved?

⑥ INSPECTION / TESTING

This can internally divide into:

Before Installation

During Installation

After Installation

Testing

Only applicable groups appear.

⑦ RECORDS / EVIDENCE

Complete objective-evidence expectations.

Not merely Quick's minimum evidence.

⑧ NC / DEFICIENCY

- common defects;

- what constitutes concern;

- escalation triggers;

- link to common Section 14 workflow.

⑨ CORRECTIVE ACTION

How the correction should be controlled from the QC perspective.

⑩ VERIFICATION

How corrected/completed work is actually verified.

⑪ CLOSURE

What must be true before accepting/closing the activity.

⑫ COMMUNICATIONS

Organized:

Before

During

Issue/Escalation

After

This preserves your original operating-system concept.

⑬ OUTPUTS

At completion, what should exist?

- records;

- evidence;

- acceptance documentation;

- follow-up actions.

⑭ REPORTING / ANALYSIS

What recurring information is worth escalating or analyzing?

This is where the inspector begins thinking like a quality manager, rather than merely inspecting individual items.

⑮ QUALITY CHECKPOINT

The final condensed self-audit.

Before I leave this activity, did I actually cover everything important?

15. FULL MODE — NAVIGATION

Full gets a persistent compact relationship strip:

BEFORE | GATES | INTERFACES | TESTING | AFTER | CLOSEOUT

Unlike Quick, Full users are deliberately investigating the workflow, so these can be slightly more prominent.

Still, they should not overwhelm the technical content.

16. FULL MODE — COLLAPSE BEHAVIOR

Very important.

Full does not mean:

1,500-word wall of text.

Instead:

▼ Quality Objective

▶ Requirements

▶ Planning

▶ Document Control

▶ Material / Equipment

▼ Inspection / Testing

▶ Records / Evidence

▶ NC / Deficiency

▶ Corrective Action

▶ Verification

▶ Closure

▶ Communications

▶ Outputs

▶ Reporting / Analysis

▶ Quality Checkpoint

The user expands what is needed.

I would allow:

EXPAND ALL

for desktop study/preparation.

But on tablet/mobile, collapsed sections are the default.

17. FULL MODE — WATCH FOR / DON'T MISS

These should still exist, but not consume another large block.

They can appear as a compact persistent summary near the top:

⚠ 6 Watch For · ❗ 2 Don't Miss

Tap to expand.

Quick emphasizes them.

Full incorporates them.

18. LEARN MODE — PURPOSE

Learn answers:

What is happening technically, why are we checking these things, and how should I reason about them?

This is particularly important because the guide isn't supposed to make you checklist-dependent.

The goal is eventually:

You see the work → understand what matters → predict where defects are likely → know what evidence matters → know when you need specialist input.

19. LEARN MODE — EXACT CONTENT

I recommend eight sections.

① WHAT IS THIS?

Plain technical explanation of the construction activity/system.

Not a textbook chapter.

Enough to orient you.

② WHY IT MATTERS

What performance or quality function does it protect?

Examples:

- structural integrity;

- water control;

- air continuity;

- fire resistance;

- maintainability;

- drainage;

- equipment performance.

③ KNOW THE TERMS

Important EN/FR terminology will eventually integrate here.

Example:

Annular space
Backer rod
Sleeve
Firestop system
Penetrant

This becomes particularly useful for your English → French transition once Build 4 is integrated.

20. LEARN — HOW GOOD WORK LOOKS

This is important.

Instead of learning only defects:

What does a properly executed condition look like?

Explain:

- correct sequence;

- expected physical condition;

- important interfaces;

- what a good installer generally leaves behind;

- what QC should be able to verify.

This creates a positive reference model in your head.

21. LEARN — COMMON FAILURES

More detailed than Watch For.

Quick:

Oversized opening

Learn:

Explains:

- what that means;

- why it matters;

- what other conditions it affects;

- why it may fall outside an approved firestop configuration;

- what QC should do when encountering it.

So:

Watch For = visual trigger.

Common Failure = understanding.

22. LEARN — WHY EACH CRITICAL CHECK MATTERS

Not every checklist item needs an essay.

But major checks should explain their reasoning.

Example:

Annular Space

Check: Verify the opening/service geometry matches the applicable approved firestop system.

Why: Firestop systems are qualified for defined configurations; substantially different geometry may not be represented by the approved condition.

That turns memorization into reasoning.

23. LEARN — PRACTICAL EXAMPLES

Use the scenarios we deliberately embedded in Build 2.

Format:

Situation

What you observe

What should concern you

How QC should reason

What happens next

This should be one of Learn Mode's strongest features.

24. LEARN — INTERFACES

Explain why the related systems interact.

Example:

Firestop ↔ Electrical

Cable/conduit creates the penetration.

Firestop ↔ Wall

The wall provides the rated assembly.

Firestop ↔ Architectural Closure

The penetration may disappear after board/finish closure.

Firestop ↔ Commissioning / Turnover

Required life-safety evidence must eventually be complete.

Here we can explain the relationship.

Quick merely links it.

Full controls it.

Learn explains it.

That is a very clean separation.

25. LEARN — SPECIALIST / AUTHORITY BOUNDARY

Where applicable:

YOUR QC ROLE

What you can inspect/control.

SPECIALIST ROLE

What requires:

- engineer;

- electrician;

- commissioning authority;

- laboratory;

- NDT technician;

- fire/life-safety specialist;

- other authorized person.

WHEN TO ESCALATE

What conditions should make you stop guessing.

This is especially valuable early in your career.

26. LEARN MODE — FROZEN STRUCTURE

ACTIVITY

QUICK | FULL | LEARN

WHAT IS THIS?

WHY IT MATTERS

KNOW THE TERMS

HOW GOOD WORK LOOKS

COMMON FAILURES

WHY THE CRITICAL CHECKS MATTER

PRACTICAL EXAMPLES

INTERFACES & SEQUENCE

SPECIALIST / AUTHORITY BOUNDARY

(where applicable)

27. THE SAME INFORMATION CAN APPEAR AT DIFFERENT DEPTHS

This needs a rule so we don't accidentally call useful progressive disclosure "duplication."

Take one technical concept:

Firestop opening configuration

QUICK

☐ Opening / annular space acceptable

FULL

Detailed inspection criteria and evidence requirements.

LEARN

Explains what annular space is, why approved system geometry matters, common mistakes and a practical example.

That is not three pieces of independent content.

It is:

one concept → three presentation depths.

That's exactly what we want.

28. INFORMATION OWNERSHIP MATRIX

This should be frozen now.

| Information | Quick | Full | Learn |
| --- | --- | --- | --- |
| Quality objective | Brief | ✓ | Explained |
| Requirements sources | Critical only | ✓ | Context |
| Planning | — | ✓ | Context where useful |
| Document control | Critical only | ✓ | — |
| Material control | Critical checks | ✓ | Explain important concepts |
| Inspection | Condensed | ✓ | Explain reasoning |
| Testing | Critical | ✓ | Explain purpose |
| Evidence | Minimum | ✓ | Explain why |
| Common deficiencies | Watch For | ✓ | Detailed |
| Corrective action | — | ✓ | Examples/reasoning |
| Verification | Critical if needed | ✓ | Explain |
| Closure | Compact | ✓ | Context |
| Communications | Critical alert only | ✓ | — |
| Outputs | Minimum | ✓ | — |
| Reporting/analysis | — | ✓ | Managerial context |
| Quality checkpoint | Core | ✓ | Explain selected items |
| Interfaces | Compact links | ✓ | Explained |
| Before/After | Compact | ✓ | Explained where useful |
| Gates | Critical | ✓ | Explain purpose |
| Watch For | ✓ | Available | Explained |
| Don't Miss | ✓ | Available | Explained |
| Practical examples | — | Optional link | ✓ |
| Terminology | Tooltip only | Tooltip | ✓ |
| Specialist boundary | Critical warning | ✓ | Explained |
| Invalidation | Warning | ✓ | Explained |

This matrix is important enough to eventually become part of the implementation specification.

29. MODE SWITCHING MUST PRESERVE CONTEXT

Suppose you're looking at:

8.9 Mechanical Penetrations → Quick

and tap:

Learn

You should land on:

8.9 Mechanical Penetrations → Learn

not the Learn homepage.

Then switch to Full:

8.9 Mechanical Penetrations → Full

Same activity, same position conceptually.

Likewise language switching eventually works:

8.9 + QUICK + EN

↓

8.9 + QUICK + FR

The underlying activity does not change.

30. DEFAULT MODE

For normal navigation:

QUICK should be the default.

Because this is ultimately supposed to become a field tool.

However, we can later allow the interface to remember the user's last mode during the session.

For example, if you're studying everything in Learn, opening another activity could remain in Learn.

That's Build 6 behavior, not Build 5 content architecture.

31. HIGH-RISK ACTIVITIES DO NOT AUTOMATICALLY FORCE FULL MODE

I would avoid that.

Instead Quick can show:

⚠ High-control activity — Full Inspection recommended

Then you decide.

The application guides; it doesn't become annoying.

32. QUICK SHOULD NEVER PRETEND TO BE COMPLETE

This should be explicitly communicated in the design.

Quick means:

Field-critical summary

not:

Every possible acceptance requirement.

Something subtle like:

Quick — Critical field checks

Full:

Full — Complete QC workflow

Learn:

Learn — Understanding & examples

That prevents someone from interpreting a seven-item Quick checklist as the entire project inspection requirement.

33. PROJECT DOCUMENTS ALWAYS GOVERN

We should reserve one subtle universal notice:

Project requirements govern. Verify applicable drawings, specifications, approved submittals and PIE/PRIE.

It doesn't need to scream from every page.

But it should always be accessible.

This maintains the boundary we've been very careful about:

our guide helps you execute QC systematically; it does not supersede the actual project QMS.

34. WHAT HAPPENS TO THE ORIGINAL INPUT → PROCESS → OUTPUT → COMMUNICATION MODEL?

It survives completely.

In Full Mode:

INPUTS

Requirements
Documents
Materials
Approved information

PROCESS

Planning
Inspection/testing
Issue control
Corrective action
Verification

OUTPUTS

Records
Evidence
Acceptance
Follow-up

COMMUNICATIONS

Before
During
Escalation
After

Quick simply extracts the field-critical pieces from that model.

Learn explains the reasoning behind it.

So your original concept remains the underlying operating philosophy.

35. 5A — FINAL PRESENTATION CONTRACT

We can now freeze the purpose of the three modes as:

QUICK

What must I pay attention to right now?

Optimized for seconds/minutes.

FULL

How do I control this activity thoroughly from requirements to closure?

Optimized for preparation and execution.

LEARN

How does this work, why does it matter, and how should a strong quality professional reason about it?

Optimized for understanding and professional development.

And across all three:

same Activity ID → same Build 2 source → same Build 3 relationships → different depth.

BUILD 5A — FROZEN ✓

We have now defined:

Quick purpose and exact blocks ✓
Quick exclusions ✓
Full purpose and exact sections ✓
Full collapse model ✓
Learn purpose and exact sections ✓
Information ownership matrix ✓
Progressive disclosure rule ✓
Mode-context preservation ✓
Default mode ✓
High-risk behavior ✓
Project-QMS boundary ✓
Input → Process → Output → Communication preservation ✓

The next step, 5B, is where this becomes especially important: we define the extraction rules that convert the enormous Build 2 material into excellent Quick Mode content without manually rewriting every page arbitrarily. That is what will make Quick Mode consistent across concrete, roofing, HVAC, electrical, firestop, finishes, earthworks, commissioning, and everything else.

Yes. 5B is the Quick-Mode Extraction Engine.

5A told us what Quick contains.
5B now tells us how the system decides what deserves to appear there.

This is important because we have a huge Build 2 knowledge base. We do not want to manually invent a short checklist for every activity based on intuition. We need consistent rules that compress the detailed content without throwing away the things that can actually hurt quality.

BUILD 5 — STEP 5B

QUICK-MODE EXTRACTION RULES

The governing principle should be:

Quick Mode is not a summary of Full Mode. It is a risk-weighted field extraction from Full Mode.

That distinction matters.

A summary tries to represent everything proportionally.

Quick should disproportionately preserve things that are:

- prerequisites;

- physically inspectable;

- critical to acceptance;

- about to become concealed;

- difficult/expensive to correct later;

- interface-sensitive;

- test-sensitive;

- evidence-sensitive;

- specialist-sensitive.

1. THE EXTRACTION PIPELINE

Every activity goes through the same process:

BUILD 2 CONTENT

↓

Canonical Activity Schema

↓

Identify Field-Critical Items

↓

Classify

↓

Score / Prioritize

↓

Remove Duplicates

↓

Compress Wording

↓

Assign to Quick Blocks

↓

Validate Against Gates + Interfaces

↓

QUICK MODE

So Quick is generated from structured knowledge, not written independently.

2. THE SIX EXTRACTION QUESTIONS

For every detailed Build 2 item, ask:

Q1 — Must this be true before work proceeds?

If yes:

→ candidate for BEFORE

Q2 — Can the inspector directly verify this in the field?

If yes:

→ candidate for INSPECT

Q3 — Could this condition become hidden, inaccessible or expensive to reconstruct?

If yes:

→ high-priority INSPECT, BEFORE, or DON'T MISS

Q4 — What evidence would be difficult/impossible to recreate later?

If important:

→ EVIDENCE

Q5 — What defect is visually common, consequential or easy to overlook?

If yes:

→ WATCH FOR

Q6 — What single mistake could undermine the whole inspection/acceptance?

If yes:

→ DON'T MISS

These six questions can extract most of Quick Mode.

3. SOURCE PRIORITY

Not all Full fields contribute equally.

I recommend this extraction priority:

Priority 1 — strongest Quick sources

- Quality Checkpoint

- Inspection/Test

- Gate requirements

- Pre-Concealment

- Closure Criteria

- Evidence

- Common Deficiencies

- Specialist Boundary

- Invalidation Rules

Priority 2

- Requirements

- Material Control

- Document Control

- Planning

- Verification

Priority 3

- Corrective Action

- Communications

- Outputs

Normally excluded

- detailed Reporting/Analysis

- long explanations

- detailed practical examples

- educational reasoning

Those belong mainly to Full/Learn.

4. QUICK ITEM CLASSIFICATION

Every extracted candidate gets classified as one of:

PREREQUISITE

FIELD_CHECK

EVIDENCE

VISUAL_RISK

CRITICAL_REMINDER

NEXT_STEP

SPECIALIST_ALERT

Then the mapping is automatic:

| Classification | Quick Block |
| --- | --- |
| PREREQUISITE | BEFORE |
| FIELD_CHECK | INSPECT |
| EVIDENCE | EVIDENCE |
| VISUAL_RISK | WATCH FOR |
| CRITICAL_REMINDER | DON'T MISS |
| NEXT_STEP | NEXT / GATE |
| SPECIALIST_ALERT | DON'T MISS / Gate warning |

This keeps the transformation predictable.

5. BEFORE — EXTRACTION RULE

An item qualifies for BEFORE if failure to establish it would make the inspection premature, unreliable or unauthorized.

Typical sources:

Requirements

Is the governing information available?

Document Control

Correct revision?

Material Control

Approved material/product?

Gate

Prerequisite satisfied?

Planning

Required witness/specialist available?

Previous work

Required upstream work accepted?

6. BEFORE — PRIORITY ORDER

When too many candidates exist, prioritize:

1. Governing requirement available

2. Upstream work accepted

3. Approved material/equipment

4. Concealed/interface prerequisites

5. Required specialist/witness

6. Environmental/site readiness

7. Administrative prerequisites

But we don't mechanically show all seven.

Usually 3–7 checks.

7. BEFORE — EXAMPLE: CONCRETE POUR

Full content may contain dozens of checks.

Quick BEFORE could become:

BEFORE

☐ Current drawings / pour requirements available
☐ Formwork accepted
☐ Reinforcement accepted
☐ Embeds / sleeves / cast-ins verified
☐ Concrete mix approved
☐ Pre-pour gate released
☐ Testing/sampling arrangements ready

Notice what happened.

We did not list every rebar check here.

Those belong in the reinforcement activity.

Concrete Pour Quick Mode is asking:

Are we actually ready to pour?

That is exactly the behavior we want.

8. INSPECT — EXTRACTION RULE

An item belongs in INSPECT when:

- it can be verified physically;

- it affects compliance/performance;

- it is appropriate to the activity;

- it is not merely explanatory;

- omission could reasonably allow defective work through.

Primary sources:

Inspection/Test + Quality Checkpoint + Closure Criteria.

9. INSPECT — COMPRESSION RULE

Detailed Build 2 checks should be grouped by inspection concept.

For example, Full may say:

- verify member location;

- verify orientation;

- verify elevation;

- verify alignment;

- verify plumbness.

Quick can say:

☐ Location / orientation / alignment / elevation / plumbness

only if grouping does not destroy field usefulness.

But we should not compress:

firestop product + annular space + installed depth + backing

into:

☐ Firestop okay

That destroys the inspection logic.

10. THE ATOMICITY RULE

This is crucial.

A Quick checkbox should be:

small enough that checking it actually means something

but:

large enough that the screen isn't ridiculous.

Bad:

☐ Installation acceptable

Too broad.

Also bad:

☐ Bolt 1
☐ Bolt 2
☐ Bolt 3...

Too granular.

Good:

☐ Bolt type / installation / completeness
☐ Required tightening verified

The correct unit is a field-verifiable quality concept.

11. INSPECT — MAXIMUM SIZE

Target:

5–10 inspection concepts

Soft maximum:

12

If we exceed 12 regularly, we should either:

- group related checks;

- split the activity;

- send secondary checks to Full.

This protects field usability.

12. CONDITIONAL CHECKS

Very important for a universal guide.

Suppose Firestop has:

identification label where required.

Quick should show:

☐ Identification / label where required

not pretend every project uses identical labeling.

Likewise:

☐ NDT where specified

☐ Primer where required

☐ Vapour barrier where applicable

The conditions from Build 3 survive compression.

13. EVIDENCE — EXTRACTION RULE

Evidence gets into Quick when one of these is true:

A. Condition will disappear

Examples:

- reinforcement;

- buried service;

- firestop;

- insulation;

- air barrier.

B. Measurement matters

Examples:

- elevation;

- thickness;

- torque/test result;

- dimensions.

C. Traceability matters

Examples:

- firestop system;

- equipment ID;

- material batch;

- test record.

D. Acceptance depends on external evidence

Examples:

- test report;

- specialist acceptance;

- certificate.

14. EVIDENCE — THREE LEVELS

We should classify evidence internally:

ESSENTIAL

Must appear in Quick.

USEFUL

Full Mode.

CONDITIONAL

Quick only when applicable.

This is better than trying to show every possible record.

15. EVIDENCE — EXAMPLE: REINFORCEMENT

Full may expect:

- inspection form;

- drawing reference;

- bar size;

- spacing;

- cover;

- lap;

- embeds;

- photos;

- location;

- deficiencies;

- corrective evidence.

Quick:

EVIDENCE

☐ Location / element ID
☐ Critical measurements where required
☐ Photos before pour
☐ Pre-pour inspection/release record

Enough to remind the inspector what must not disappear.

16. WATCH FOR — EXTRACTION RULE

This should come primarily from:

Common Deficiencies + practical examples + recurring interface failures.

A defect qualifies if it is:

- visually recognizable;

- reasonably common;

- consequential;

- easy to overlook;

- associated with later rework/failure.

17. WATCH FOR SHOULD BE PHYSICAL LANGUAGE

Avoid:

non-compliant installation

That tells you nothing.

Prefer:

displaced rebar
torn membrane
missing firestop
wet insulation
unsupported conduit
reversed equipment orientation
blocked access
damaged coating

The user should be able to look up from the tablet and hunt for the condition.

That's the point.

18. WATCH FOR — LIMIT

Target:

3–6

Maximum:

8 only for unusually complex activities.

No explanations in Quick.

Tap item → Learn can eventually explain it.

19. DON'T MISS — EXTRACTION RULE

An item qualifies for Don't Miss when omission has disproportionate consequences.

I recommend five triggers.

Trigger A — Irreversible/concealed

Verify before covering.

Trigger B — Whole-system dependency

Example:

air-barrier continuity at penetration.

Trigger C — Authority boundary

Example:

electrical specialist acceptance.

Trigger D — Gate dependency

Example:

pre-pour release.

Trigger E — Traceability dependency

Example:

capture system ID before concealment.

20. DON'T MISS — RANKING

If several candidates exist:

Rank 1

Safety/life-safety or specialist boundary.

Rank 2

Concealment/irreversibility.

Rank 3

Major performance/interface risk.

Rank 4

Test/gate requirement.

Rank 5

Critical evidence.

Then select:

1–3 maximum.

21. DON'T MISS MUST NOT BECOME WATCH FOR

They have different jobs.

WATCH FOR

What defects should my eyes actively hunt for?

DON'T MISS

What must my brain absolutely remember before I leave/release this work?

Example roof:

WATCH FOR

- punctures

- wrinkles

- bad seams

- wet substrate

- damaged flashing

DON'T MISS

Inspect penetrations and membrane condition before later equipment/traffic obscures them.

Excellent separation.

22. NEXT / GATE — EXTRACTION RULE

This comes almost entirely from Build 3.

Quick asks:

What downstream activity becomes possible if this condition is accepted?

and:

Is there a gate between here and there?

Example:

Foundation Reinforcement

NEXT

→ Concrete Placement

GATE

⚠ G-STR-01 Pre-Pour Release

23. MULTIPLE DOWNSTREAM ACTIVITIES

Do not dump six links.

Show the most immediate:

Next: Wall Closure

Then:

+3 related downstream works

under Related.

This keeps Quick directional.

24. INTERFACE EXTRACTION

Quick should not show every INTERFACES_WITH.

We need a relevance filter.

Show an interface in Quick if:

- it is physically present at the inspection location;

- it affects acceptance;

- it can become concealed;

- it can invalidate another system;

- coordination failure is common/high consequence.

Other interfaces remain under Related/Full.

25. INTERFACE PRIORITY

Rank:

1. Physical interface

Two systems literally meet.

2. Concealment interface

One trade can hide another.

3. Performance interface

One affects performance of another.

4. Access interface

One can obstruct another.

5. Sequence-only interface

Lower Quick priority.

26. INVALIDATION EXTRACTION

Step 6 now becomes useful.

If the activity has an important invalidation rule, Quick can show a contextual reminder.

Example:

Firestop

❗ Reinspect if the penetration is modified later.

Pressure-tested piping

❗ Repair/modification after testing may require retest review.

Roof membrane

❗ Later penetrations/damage can reopen acceptance.

Not every invalidation rule appears.

Only high-value ones.

27. SPECIALIST-BOUNDARY EXTRACTION

If:

specialistBoundary.required = true

then Quick must preserve the boundary whenever it materially affects acceptance.

Example:

ELECTRICAL TESTING

❗ DON'T MISS

Verify applicable PIE/PRIE and required acceptance by authorized electrical personnel.

We must never compress specialist-sensitive content so aggressively that Quick accidentally implies:

“You personally determine technical electrical acceptance.”

28. GATE EXTRACTION

If an activity is controlled by a hard gate, Quick always surfaces it.

Conditional gates appear when relevant:

Gate: G-INT-01 — before wall closure

If there is no meaningful gate:

don't show an empty Gate box.

29. PRE-CONCEALMENT OVERRIDE

This deserves a special rule.

If:

preConcealment.applies = true

then Quick extraction increases the priority of:

- hidden-condition inspection;

- photos;

- measurements;

- unresolved issues;

- gate release;

- affected interfaces.

In other words:

Pre-concealment status automatically biases Quick toward evidence and completeness.

That's exactly what a field inspector needs.

30. HIGH-CONTROL OVERRIDE

If an activity is tagged:

highControl

we do not make Quick enormous.

Instead:

⚠ High-control activity

and potentially:

Full Inspection recommended

The Quick checklist remains concise.

This is important.

Complexity should not destroy our presentation model.

31. ISSUE-STATE OVERRIDE

Suppose you open an activity because something is wrong.

Quick should surface:

Issue? → Open Deficiency / NC Workflow

Not give the entire Section 14 process.

Likewise:

Failed test? → Failed-Test Workflow

This leverages Build 3 rather than bloating Quick.

32. DUPLICATE ELIMINATION

Suppose:

Quality Checkpoint
says:

verify firestop depth.

Inspection
says:

measure installed sealant depth.

Closure
says:

required depth acceptable.

Quick should contain:

☐ Required installed depth

once.

Full preserves the detailed context in the relevant sections.

This deduplication is fundamental.

33. SEMANTIC MERGING

The extractor should recognize concepts, not merely identical sentences.

For example:

verify membrane is continuous

and

no gaps in air barrier

may become:

☐ Membrane continuity / no gaps

But only when they genuinely represent the same quality concept.

We should be conservative.

34. DO NOT MERGE DIFFERENT ACCEPTANCE CONCEPTS

For example:

membrane continuity

and:

membrane adhesion

are not the same.

Likewise:

equipment location

and:

maintenance clearance

are not the same.

So compression must preserve technical meaning.

35. LANGUAGE STYLE FOR QUICK

Quick should use:

noun phrases + action-oriented technical wording

Examples:

Good:

☐ Approved product/system
☐ Location / elevation
☐ Supports / spacing
☐ Penetration sealing
☐ Required test complete

Avoid:

☐ Make sure that the product being used by the subcontractor is the correct product that has previously been approved...

That's Full/Learn territory.

36. CHECKBOXES SHOULD REPRESENT VERIFICATION, NOT ACTION

Prefer:

☐ Correct material / model

rather than:

☐ Check the material.

The checkbox itself already means check.

This saves words everywhere.

37. USE MEASUREMENTS WHEN THE CONCEPT REQUIRES THEM

Do not dumb technical checks down.

If Build 2 says the inspector needs to verify:

- elevation;

- thickness;

- spacing;

- slope;

- cover;

- dimensions;

- clearance;

Quick should preserve the measurement concept.

Example:

☐ Cover / spacing / lap dimensions

not:

☐ Rebar looks correct.

38. DO NOT INVENT VALUES

Critical universal-guide rule.

Quick may say:

☐ Required concrete cover

It should not say:

☐ 50 mm cover

unless that value comes from the applicable project requirement we are actually using.

Similarly:

required slope
specified thickness
approved torque
required test pressure

rather than fabricated universal numbers.

This keeps the guide safe and reusable.

39. DO NOT INVENT HOLD POINTS

Same principle.

If the universal framework says a gate is conceptually important, we can say:

Verify applicable PIE/PRIE hold/witness requirements.

But we don't pretend CRT/Hydro-Québec formally labels something as a hold point unless their project documents actually do.

Our gate architecture is our navigation/control logic, not a replacement for contractual PIE terminology.

40. CONDITIONALITY MUST SURVIVE QUICK MODE

Never compress:

where required

out of existence.

Examples:

☐ Primer where required
☐ NDT where specified
☐ Firestop where rated
☐ Specialist acceptance where required

This prevents false universality.

41. QUICK EXTRACTION SCORING MODEL

We don't necessarily need to expose this in the application, but I would define it for consistency.

Each candidate can conceptually receive points:

| Criterion | Weight |
| --- | --- |
| Concealed/irreversible | +5 |
| Gate prerequisite | +5 |
| Safety/life-safety | +5 |
| Specialist acceptance | +5 |
| Critical performance | +4 |
| High interface risk | +4 |
| Required test | +4 |
| Difficult rework | +3 |
| Critical evidence | +3 |
| Common field defect | +3 |
| Easily visually verified | +2 |
| Mainly explanatory | −4 |
| Mainly administrative | −2 |
| Duplicate concept | remove |

This doesn't need to become complicated code immediately.

It is a content-curation rule.

42. QUICK EXTRACTION EXAMPLE — AIR BARRIER

Suppose Full contains extensive information on:

- drawings;

- manufacturer requirements;

- substrate;

- product approval;

- compatibility;

- primer;

- membrane installation;

- laps;

- seams;

- transitions;

- penetrations;

- openings;

- damage;

- repairs;

- testing;

- records;

- deficiencies.

Quick becomes approximately:

BEFORE

☐ Approved air-barrier system
☐ Substrate acceptable
☐ Required details/transitions identified
☐ Compatible accessories/sealants

INSPECT

☐ Membrane continuity
☐ Laps / seams
☐ Transitions
☐ Openings
☐ Penetrations
☐ Terminations
☐ Damage / repairs

EVIDENCE

☐ Area/location
☐ Critical transitions/penetrations photographed
☐ Pre-concealment condition recorded

⚠ WATCH FOR

- tears/punctures

- open seams

- incomplete transitions

- unsealed penetrations

- incompatible repair

- later-trade damage

❗ DON'T MISS

Continuity at transitions and penetrations.

NEXT

→ Insulation / cladding / concealment as applicable.

Notice how much technical information survives without turning Quick into Full.

43. QUICK EXTRACTION EXAMPLE — HVAC EQUIPMENT

BEFORE

☐ Approved equipment/model
☐ Location/base ready
☐ Required clearances established
☐ Connections/interfaces coordinated

INSPECT

☐ Location / orientation
☐ Anchorage / support
☐ Service clearances
☐ Vibration isolation where required
☐ Duct / pipe / drain connections
☐ Electrical/control interfaces
☐ Identification

EVIDENCE

☐ Equipment ID
☐ Installation photos
☐ Relevant inspection/start-up records

⚠ WATCH FOR

- wrong orientation

- blocked access

- poor anchorage

- missing isolation

- misaligned connections

- damaged equipment

❗ DON'T MISS

Confirm maintainability and removal/service access before surrounding work makes correction difficult.

NEXT

→ Connections → Start-Up → Functional Testing → Commissioning.

That is excellent field-level compression.

44. QUICK EXTRACTION EXAMPLE — STRUCTURAL STEEL ERECTION

BEFORE

☐ Approved shop/erection drawings
☐ Correct members identified
☐ Supports/anchors ready
☐ Erection sequence/bracing requirements understood

INSPECT

☐ Member ID / location / orientation
☐ Elevation / alignment / plumbness
☐ Temporary bracing
☐ Connections complete
☐ Damage / field modifications

EVIDENCE

☐ Member/location identification
☐ Relevant measurements
☐ Connection/inspection records where required

⚠ WATCH FOR

- wrong member/orientation

- misalignment

- missing/incomplete connection

- inadequate temporary bracing

- unauthorized field modification

- coating damage

❗ DON'T MISS

Field modifications require appropriate technical authorization.

Again: concise, but not simplistic.

45. QUICK EXTRACTION EXAMPLE — PRESSURE TEST

This shows that the same schema works for testing.

BEFORE

☐ Correct system/test boundary identified
☐ Installation ready for test
☐ Current procedure/acceptance criteria available
☐ Instruments/calibration acceptable
☐ Required personnel/witness available

INSPECT

☐ Test configuration
☐ Required test conditions
☐ Instrument readings
☐ Leakage / pressure behavior
☐ Result documented

EVIDENCE

☐ System/test ID
☐ Test readings
☐ Test report
☐ Required witness/acceptance

⚠ WATCH FOR

- wrong test boundary

- incomplete isolation

- uncalibrated instrument

- leakage

- undocumented result

❗ DON'T MISS

A failed test stays traceable through correction, retest and acceptance.

NEXT

PASS → downstream work/acceptance

FAIL → Failed-Test Management

The model survives even when there isn't a traditional physical “installation inspection.”

46. QUICK EXTRACTION EXAMPLE — FINAL DEFICIENCY WALK

BEFORE

☐ Area sufficiently complete
☐ Current drawings/requirements available
☐ Existing open items reviewed

INSPECT

☐ Workmanship / finish
☐ Damage
☐ Completeness
☐ Operation/access where applicable
☐ MEP/architectural interfaces
☐ Previously corrected items

EVIDENCE

☐ Exact location
☐ Clear deficiency description
☐ Photo
☐ Responsible trade/category

⚠ WATCH FOR

- repeat defects

- concealed unresolved work

- damage from later trades

- incomplete corrections

- inaccessible equipment

❗ DON'T MISS

“Done” is not “closed” until correction is verified.

NEXT

→ Assignment → Correction → Reinspection → Closure.

So Quick works for Section 14 too.

47. QUICK MODE SHOULD BE DYNAMIC BY ACTIVITY TYPE

Same architecture.

Different emphasis.

Physical installation

INSPECT dominates.

Testing

BEFORE + result/evidence dominate.

Interface

coordination + restoration dominate.

Closeout

status + evidence + closure dominate.

Gate

prerequisites + blockers dominate.

Therefore we should not force identical visual proportions on every page.

The blocks remain familiar; their content weight adapts.

48. QUICK FOR SPECIALIZED GATE PAGES

A gate Quick page should instead show:

READY WHEN

☐ prerequisite
☐ prerequisite
☐ prerequisite

BLOCK IF

- unresolved deficiency

- missing inspection

- missing test

- missing specialist release

EVIDENCE

☐ required records

RELEASES

→ downstream activity

❗ DON'T MISS

critical release condition.

That is better than pretending a gate is a normal construction activity.

49. QUICK FOR INTERFACE PAGES

Example 11.7 Penetrations:

BEFORE

☐ Opening/service coordinated
☐ Assembly requirements identified

CHECK THE INTERFACE

☐ Opening/sleeve
☐ Service
☐ Structural condition
☐ Air/water control
☐ Insulation
☐ Firestop
☐ Finish

EVIDENCE

☐ Location
☐ Before/after photos
☐ Required system references

WATCH FOR

- oversized opening

- damaged structure

- unsealed interface

- missing insulation restoration

- missing firestop

DON'T MISS

The penetration is not complete until the entire assembly is restored.

Very useful.

50. QUICK VALIDATION TEST

Before any Quick page is accepted into the final dataset, we should ask:

Test 1 — Field speed

Can I understand it in 10–20 seconds?

Test 2 — Actionability

Does every checkbox tell me something I can actually verify?

Test 3 — Criticality

Did we preserve the most consequential controls?

Test 4 — Concealment

Did we preserve what could disappear?

Test 5 — Evidence

Did we preserve evidence that cannot easily be recreated?

Test 6 — Interfaces

Did we preserve critical cross-trade risks?

Test 7 — Authority

Did we preserve specialist boundaries?

Test 8 — Universality

Did we avoid invented project-specific criteria?

Test 9 — Brevity

Did Full Mode accidentally leak into Quick?

Test 10 — Traceability

Can every Quick item be traced back to Build 2/Build 3?

If any answer is no, the extraction needs revision.

51. TRACEABILITY IS IMPORTANT

Every Quick item should internally know its source.

Conceptually:

quickCheck {

text

category

sourceFields[]

sourceActivityId

condition

priority

}

Example:

text:

"Penetrations / transitions sealed"

sourceFields:

inspection

qualityCheckpoint

sourceActivityId:

4.2

priority:

critical

This becomes very valuable when we eventually edit the knowledge base.

If Build 2 changes, we know what Quick content depends on it.

52. QUICK CONTENT SHOULD NOT BE AUTOMATICALLY GENERATED AT RUNTIME

Important implementation decision.

I would not have the finished app dynamically ask an AI:

“Summarize this activity into Quick Mode.”

That creates inconsistency.

Instead:

Build 2 → extraction rules → curated Quick dataset → application.

AI/Codex can help us create it during development.

But once approved, Quick content becomes deterministic structured data.

That means:

- same checklist every time;

- offline capability;

- fast loading;

- predictable translation;

- easy quality review.

Much better for this use case.

53. TRANSLATION COMES AFTER EXTRACTION

This is another efficiency win.

We should not independently shorten English and French Build 2 content.

Instead:

English Build 2

↓

Canonical technical concepts

↓

Approved Quick extraction

↓

Quick EN

↓

Build 4 terminology/translation

↓

Quick FR

This keeps the EN/FR versions semantically aligned.

54. FINAL QUICK-MODE EXTRACTION ALGORITHM

For each activity:

STEP A

Load:

- Quality Checkpoint

- Inspection/Test

- Gate requirements

- Pre-Concealment

- Closure

- Evidence

- Common Deficiencies

- Specialist Boundary

- Invalidation Rules

STEP B

Extract candidate concepts.

STEP C

Classify:

- prerequisite;

- field check;

- evidence;

- visual risk;

- critical reminder;

- next step;

- specialist alert.

STEP D

Score criticality.

STEP E

Preserve applicable conditions.

STEP F

Semantically deduplicate.

STEP G

Compress wording without destroying technical meaning.

STEP H

Apply limits:

Before: ~3–7
Inspect: ~5–10
Evidence: ~2–5
Watch For: ~3–6
Don't Miss: 1–3

STEP I

Add Build 3:

- immediate gate;

- most relevant interfaces;

- immediate downstream activity;

- invalidation warning where critical.

STEP J

Run the 10-point Quick validation test.

STEP K

Freeze the resulting Quick object.

55. RESULTING DATA OBJECT

Something like:

QuickView {

activityId

before[]

inspect[]

evidence[]

watchFor[]

dontMiss[]

gate {

gateId

message

}

next {

primaryActivityId

additionalCount

}

related {

priorityRelationshipIds[]

}

specialistAlert

invalidationAlert

qcThinkEnabled

}

Simple.

The complexity remains underneath.

56. THE REAL VALUE OF 5B

This is what prevents our system from becoming either extreme:

Extreme 1

A giant technical manual that is excellent academically but annoying on site.

Extreme 2

A simplistic checklist app that says:

“Check concrete ✓”

and teaches you almost nothing.

Instead we get:

Deep technical knowledge underneath + aggressive but intelligent field compression on top.

And because the compression prioritizes gates, concealment, interfaces, evidence, common failures, specialist boundaries and invalidation, Quick Mode should actually reinforce good QC habits rather than merely save screen space.

BUILD 5B — FROZEN ✓

We now have:

Extraction pipeline ✓
Candidate classification ✓
Source priority ✓
Risk-weighting logic ✓
Before extraction ✓
Inspect extraction ✓
Atomicity rule ✓
Evidence extraction ✓
Watch For extraction ✓
Don't Miss extraction ✓
Gate/Next extraction ✓
Interface filtering ✓
Invalidation extraction ✓
Specialist-boundary preservation ✓
Pre-concealment override ✓
High-control override ✓
Semantic deduplication ✓
Conditionality preservation ✓
No invented values/hold points ✓
Specialized activity handling ✓
10-point validation test ✓
Source traceability ✓
Deterministic curated output ✓
EN→FR extraction order ✓

Next is 5C — Full Mode Presentation. We already know what Full contains from 5A; 5C should now define how those enormous activity workflows are organized, collapsed, cross-linked and navigated so that Full Mode remains thorough without becoming another giant Build 2 document on a screen.

Yes. 5C is Full Mode Presentation.

5A defined what Full contains.
5B defined how Quick is intelligently compressed.
Now 5C answers:

How do we expose the full Build 2 workflow without recreating an 847-page manual inside every activity screen?

The answer is progressive disclosure, structured navigation, and persistent context.

Build 2 already gives us the recurring full-control backbone—Requirements, Planning, Document Control, Material Control, Inspection/Test, Records, NC/Deficiency, Corrective Action, Verification, Closure, Reporting/Analysis, and Quality Checkpoint.

BUILD 5 — STEP 5C

FULL MODE PRESENTATION

1. Core purpose

Full Mode answers:

How do I control this activity properly from requirements through closure?

It is for:

- inspection preparation;

- unfamiliar activities;

- high-risk work;

- complex interfaces;

- issue resolution;

- pre-concealment planning;

- testing preparation;

- checking whether anything was missed.

It should feel like a professional QC operating procedure, not a textbook.

2. Full Mode should not be one giant page

The biggest design failure would be:

open Full → see 2,000 words.

Instead, every section is a collapsible card.

Default state:

QUALITY OBJECTIVE            ▼

REQUIREMENTS                 ▶

PLANNING                     ▶

DOCUMENT CONTROL             ▶

MATERIAL / EQUIPMENT         ▶

INSPECTION / TESTING         ▼

RECORDS / EVIDENCE           ▶

NC / DEFICIENCY              ▶

CORRECTIVE ACTION            ▶

VERIFICATION                 ▶

CLOSURE                      ▶

COMMUNICATIONS               ▶

OUTPUTS                      ▶

REPORTING / ANALYSIS         ▶

QUALITY CHECKPOINT           ▶

On first opening Full Mode, I would default to:

- Quality Objective open

- Inspection / Testing open

Everything else collapsed.

That immediately exposes the purpose and core field process without overwhelming you.

3. Persistent Full header

At the top:

8.9 — Mechanical Penetrations

QUICK | FULL | LEARN

Then a compact navigation strip:

Before · Gates · Interfaces · Testing · After · Closeout

And small indicators when applicable:

⚠ Pre-Concealment
◇ High-Control
⚡ Specialist Interface

These indicators should be tappable, not giant warnings.

4. Quality Objective

Always short and open by default.

Example:

QUALITY OBJECTIVE

Ensure the penetration is properly located, structurally coordinated, correctly sleeved/sealed, and that the affected fire, air, water, insulation and finish systems are restored.

Build 2 treats mechanical penetrations explicitly as a multidisciplinary condition involving mechanical, structural, architectural/envelope and fire/life-safety requirements.

This tells you why the page exists before you dive into details.

5. Requirements card

REQUIREMENTS

This should be grouped by source type, not dumped as a flat list.

Example:

Drawings

- mechanical drawings;

- structural opening drawings;

- architectural details.

Approved Technical Information

- approved coordination drawings;

- sleeve details;

- firestop system;

- waterproofing detail;

- air-barrier detail.

Quality / Procedure

- PIE/PRIE;

- applicable inspection/test requirements.

That structure mirrors the Build 2 source while making retrieval faster.

At the bottom:

Project documents govern.

No invented criteria.

6. Planning card

PLANNING

Use short subheadings:

Scope

- identify area/system/component.

Sequence

- determine what must occur before and after.

Interfaces

- identify affected disciplines.

Inspection Timing

- before cutting;

- before installation;

- before concealment;

- after completion.

Testing

- identify required tests/witnesses.

Evidence

- identify conditions that will disappear.

Planning should answer:

What do I need organized before I go inspect this?

7. Document Control card

This should be compact and highly standardized across all activities.

DOCUMENT CONTROL

☐ Latest drawing/revision
☐ Correct detail/shop drawing
☐ Approved technical changes incorporated
☐ Correct material/equipment reference
☐ Superseded information excluded
☐ Required approval/signatures available

We do not need prose unless the activity has a special document-control issue.

This keeps Full consistent.

8. Material / Equipment Control card

Again, use groups.

MATERIAL / EQUIPMENT CONTROL

Approval

- approved product/system/model.

Identification

- type;

- grade;

- model;

- batch where relevant.

Condition

- no damage;

- correct storage;

- shelf life where relevant.

Compatibility

- adjoining systems/materials compatible.

For closeout activities this card can adapt to equipment-record matching rather than physical material inspection. Build 2 does this for O&M and warranties by checking that manuals, models and final installed products correspond.

9. Inspection / Testing card

This is the most important card.

It should support four optional subsections:

BEFORE INSTALLATION

DURING INSTALLATION

AFTER INSTALLATION

TESTING

Only show the ones relevant to the activity.

For example:

Air Barrier

Before

- substrate;

- materials;

- transitions identified.

During

- membrane installation;

- seams/laps;

- corners;

- penetrations.

After

- continuity;

- damage;

- repairs.

Testing

- applicable envelope test where specified.

This is far more usable than one enormous undifferentiated inspection list.

10. Nested inspection groups

Some activities need another level.

Example HVAC Equipment:

INSPECTION / TESTING

Receiving

- correct equipment;

- model;

- damage.

Base / Support

- location;

- anchors;

- vibration isolation.

Installation

- orientation;

- clearances;

- access.

Connections

- duct;

- piping;

- drain;

- electrical;

- controls.

Testing / Start-Up

- applicable test/start-up evidence.

So Full Mode can have nested subgroups, but only one nesting level.

No menus inside menus inside menus.

11. Evidence card

RECORDS / EVIDENCE

Group by what proves the inspection:

Identification

- location;

- component/system ID.

Documents

- drawing/detail;

- approval reference.

Measurements

- dimensions/elevation/test values.

Photos

- overall;

- detail;

- pre-concealment.

Acceptance Evidence

- test reports;

- specialist records;

- inspection form.

This is where Full goes beyond Quick's 2–5 critical evidence reminders.

12. NC / Deficiency card

NC / DEFICIENCY

Split into two parts:

Common Deficiencies

- activity-specific problems.

Escalate When

- condition exceeds normal field correction;

- approved detail/system no longer applies;

- structural/fire/life-safety issue;

- required technical disposition needed;

- repeated test failure;

- specialist acceptance required.

Then:

Open Issue Workflow →

This links to Section 14.

We do not reproduce the whole deficiency lifecycle here.

13. Corrective Action card

This should focus on control, not invent technical repair methods.

CORRECTIVE ACTION

- Identify affected requirement.

- Control affected work.

- Document the condition.

- Determine whether approved repair/disposition is required.

- Correct using authorized method.

- Restore adjacent systems.

- Reinspect.

- Retest where required.

- Retain evidence.

Build 2's corrective-work section explicitly requires the full affected assembly to be restored; the repair itself can introduce a new deficiency.

For activity-specific examples, we can add them below this common workflow.

14. Verification card

VERIFICATION

This answers:

What must I actually recheck after installation/correction?

Typical structure:

Physical

- corrected condition.

Interfaces

- adjacent systems restored.

Measurement

- dimensions/elevation/thickness/etc.

Test

- required test/retest.

Technical Acceptance

- specialist/professional acceptance where required.

Evidence

- final record complete.

This card should remain separate from Corrective Action.

15. Closure card

This is the formal “am I really done?” section.

CLOSURE

☐ Physical work acceptable
☐ Required interfaces complete
☐ Required tests complete
☐ Deficiencies resolved
☐ Specialist acceptance obtained where required
☐ Evidence complete
☐ Downstream work can proceed

Activity-specific criteria supplement this.

For roof drains, for example, Build 2 requires correct drain/elevation, complete connection, membrane integration, correct sump, functional drainage and completed testing before closure.

16. Communications card

This should be one of Full Mode's most practical sections.

COMMUNICATIONS

Before

- inspection notice;

- required participants;

- required trade/specialist.

During

- immediate field coordination;

- unresolved interface questions.

Issue / Escalation

- responsible trade;

- Responsable qualité;

- relevant specialist/professional;

- superintendent/project team where applicable.

After

- inspection result;

- deficiency/NC;

- test result;

- required follow-up;

- release/closure status.

No project personnel names.

Universal roles only.

17. Outputs card

OUTPUTS

At the end of the activity, what should exist?

For example:

Records

- inspection record;

- measurements;

- photos.

Acceptance

- test report;

- release;

- specialist evidence where required.

Follow-Up

- deficiency;

- NC;

- retest;

- as-built item.

This satisfies the original Inputs → Process → Outputs → Communications concept.

18. Reporting / Analysis card

This should remain concise.

REPORTING / ANALYSIS

Look for:

- recurring deficiency type;

- repeated trade issue;

- repeated test failures;

- repeated interface conflict;

- supplier/material trend;

- documentation gaps;

- rework pattern.

Then perhaps:

Management Insight: repeated missed embeds may indicate coordination breakdown rather than isolated workmanship.

Build 2 explicitly uses reporting/analysis in this way—for example, missed embeds are treated as a possible coordination problem.

That is where Full Mode starts building QC-manager thinking.

19. Quality Checkpoint card

This should be the last card.

QUALITY CHECKPOINT

It is Build 2's final self-audit.

Example embedded items:

☐ Structural
☐ Mechanical
☐ Electrical
☐ Architectural
☐ Equipment
☐ Location
☐ Elevation
☐ Orientation
☐ Secure
☐ Pre-pour evidence

That is already the exact compact logic Build 2 uses.

In Full Mode it serves as:

“Before I leave, did I cover the activity end-to-end?”

Quick uses a distilled version of it.

20. Sticky section index

For long Full pages, especially on tablet/desktop, add a small button:

☰ Sections

Tap:

- Requirements

- Planning

- Documents

- Materials

- Inspection

- Evidence

- Issues

- Corrective Action

- Verification

- Closure

- Communications

- Outputs

- Reporting

- Checkpoint

Tap one → jump directly there.

This prevents endless scrolling.

21. Expand / Collapse controls

At top of Full:

Expand All
Collapse All

Useful distinction:

On tablet/mobile

Default mostly collapsed.

On PC

We may allow last-used state or more sections open.

But Build 6 handles persistence.

22. Cross-links inside Full

Cross-links should appear where context makes them useful, not only at the top.

Example inside Mechanical Penetrations:

Firestop

↔ 10.3 Firestopping

Envelope

↔ 4.2 Air Barrier

Universal

↔ 11.7 Universal Penetrations

This means you don't need to scroll back to the top every time a detailed inspection item references another system.

But direct inline links should be sparse.

23. Context link rule

Only insert an inline cross-link when:

- understanding the current check genuinely requires the related page;

- there is an important interface;

- another workflow owns detailed control.

Do not turn every noun into a hyperlink.

That becomes distracting.

24. Conditional sections

If a section doesn't apply, hide it.

Examples:

A purely visual architectural finish may not have meaningful commissioning.

A document-closeout node may have no literal material installation.

A test node may not need “During Installation.”

Do not show:

Material Control — N/A
Commissioning — N/A
Pre-Concealment — N/A

everywhere.

That creates clutter.

25. Conditional items remain visible with qualifiers

Different from hiding entire irrelevant sections.

If an inspection item is relevant only under certain conditions:

☐ Firestop where rated
☐ Primer where required
☐ NDT where specified

Keep it visible because it helps you ask:

Does that condition apply here?

26. Full Mode and gates

If the activity is GATED_BY, Full should show a prominent but compact gate card near the top.

Example:

GATE — WALL / CEILING PRE-CONCEALMENT

Status logic

- framing

- insulation

- MEP rough-ins

- firestop

- tests

- access

- evidence

Open Gate →

The actual gate page owns the full gate checklist.

Do not duplicate it into the activity page.

27. Full Mode and testing

Likewise:

TESTING

Related

- Mechanical Testing

- Functional Testing

Open Testing Workflow →

The activity page can describe activity-specific preparation/checks.

Section 13 owns the full test/failure/retest lifecycle.

28. Full Mode and deficiencies

Same architecture.

Activity page:

NC / DEFICIENCY

Shows:

- common defects;

- escalation triggers.

Then:

Deficiency Workflow →
NC Workflow →

Section 14 owns lifecycle.

This is one of our strongest anti-duplication rules.

29. Full Mode and as-builts

Where Build 3 says AS_BUILT_FEEDS:

Under Outputs or Closeout:

AS-BUILT IMPACT

Changes to final installed location/configuration may require record-drawing update.

Link:

14.10 Redlines / As-Builts →

No project tracking required.

30. Full Mode specialist boundary

This deserves a standardized visual card.

⚠ SPECIALIST / AUTHORITY BOUNDARY

QC Inspector

- physical installation;

- location;

- support;

- interface;

- documentation;

- required evidence.

Authorized Specialist

- specialized technical testing;

- electrical technical acceptance;

- engineered disposition;

- system programming;

- professional sign-off.

The exact content varies by activity.

This should appear near Inspection/Testing, not buried at the end.

31. Full Mode invalidation notice

Only show where meaningful.

Example:

⚠ RECHECK IF MODIFIED

Previous acceptance may need review if:

- a new penetration is added;

- work is damaged;

- repaired;

- relocated;

- tested configuration changes.

This comes from Step 6.

It should not dominate Full Mode, but it is valuable.

32. Full Mode should preserve Build 2 detail

Important rule:

Full is where detail goes; don't over-compress it just because Quick exists.

If Build 2 distinguishes:

- location;

- elevation;

- orientation;

- projection;

- anchorage;

- interference;

- secure fixing;

Full preserves them.

Quick may consolidate intelligently.

Full shouldn't silently throw away technical distinctions.

33. But Full can improve formatting

Preserving detail does not mean preserving Build 2 formatting literally.

We can reorganize:

Original Build 2 prose/list

↓

structured headings

↓

checkboxes / bullets / subgroups

without changing technical meaning.

This is presentation transformation, not content alteration.

34. Full Mode example — Mechanical Penetrations

The page might look roughly like:

8.9 MECHANICAL PENETRATIONS

QUICK | FULL | LEARN

⚠ Pre-Concealment · ◇ High-Control

Before · Gate · Interfaces · Testing · After

QUALITY OBJECTIVE ▼

Restore structural, fire, air, water, insulation and finish performance around the penetration.

REQUIREMENTS ▶

Mechanical · Structural openings · Architectural details · Firestop · Waterproofing · Air barrier · Roof · Sleeves · Coordination · PIE/PRIE

PLANNING ▶

DOCUMENT CONTROL ▶

MATERIAL CONTROL ▶

INSPECTION / TESTING ▼

Opening
☐ Location
☐ Size
☐ Structural authorization

Sleeve / Service
☐ Sleeve
☐ Clearance
☐ Service configuration

Assembly Restoration
☐ Firestop where rated
☐ Air/water seal where envelope
☐ Insulation continuity
☐ Finish closure

RECORDS / EVIDENCE ▶

NC / DEFICIENCY ▶

CORRECTIVE ACTION ▶

VERIFICATION ▶

CLOSURE ▶

COMMUNICATIONS ▶

OUTPUTS ▶

REPORTING / ANALYSIS ▶

QUALITY CHECKPOINT ▶

This is thorough without feeling like a giant Word document.

35. Full Mode example — Equipment Start-Up

Because node types vary, the structure adapts.

13.7 EQUIPMENT START-UP

Quality Objective

Requirements

Planning

Document Control

Equipment Readiness

Pre-Start Checks

Start-Up / Testing

Results / Evidence

Failure Management

Corrective Action

Retest / Verification

Acceptance

Communications

Outputs

So the schema remains consistent, but labels can adapt to node type.

36. Specialized Full templates

We therefore freeze four variants.

A. Standard Activity

Physical installation.

B. Interface Activity

Cross-system work.

C. Testing Activity

Readiness/test/result/failure/retest/acceptance.

D. Closeout Activity

Status/records/correction/verification/acceptance.

And Gate pages use a fifth:

E. Gate

Inputs/checks/blockers/release/controls/invalidation.

They share visual design.

37. Standard Activity Full order

Freeze:

- Quality Objective

- Requirements

- Planning

- Document Control

- Material / Equipment Control

- Inspection / Testing

- Records / Evidence

- NC / Deficiency

- Corrective Action

- Verification

- Closure

- Communications

- Outputs

- Reporting / Analysis

- Quality Checkpoint

38. Interface Full order

For Section 11 / interface-heavy nodes:

- Quality Objective

- Systems Involved

- Requirements

- Sequence / Coordination

- Document Control

- Materials / Components

- Interface Inspection

- Pre-Concealment

- Evidence

- Issues

- Correction

- Verification

- Closure

- Communications

- Quality Checkpoint

39. Testing Full order

- Quality Objective

- Test Requirements

- Test Readiness

- Procedure / Documentation

- Instruments / Personnel

- Test Execution

- Result

- Records / Evidence

- Failure Management

- Corrective Action

- Retest

- Specialist Acceptance

- Closure

- Communications

- Quality Checkpoint

Build 2's acceptance-evidence logic requires that the complete chain from inspection through test, failure, corrective action, retest and final acceptance be reconstructable.

40. Closeout Full order

- Quality Objective

- Requirements

- Planning

- Document Control

- Review / Inspection

- Records

- Open Items / Deficiencies

- Corrective Action

- Verification

- Closure

- Reporting / Analysis

- Quality Checkpoint

This remains close to Build 2's closeout modules.

41. Gate Full order

- Purpose

- Scope

- Inputs / Prerequisites

- Checks

- Blocking Conditions

- Specialist / External Release

- Evidence

- Release Condition

- Downstream Work Controlled

- Invalidation / Reopen Conditions

Very compact.

42. Reading state

A useful but non-essential behavior:

When you expand:

Inspection / Testing

then switch briefly to Quick and back to Full, the page can remember which Full sections were open during that session.

That belongs to Build 6 implementation polish, but our architecture should allow it.

43. Mobile/tablet control

On tablet/mobile:

- one-column layout;

- large accordion headers;

- generous tap targets;

- no tiny nested arrows;

- sticky top mode selector;

- section-jump button.

On PC:

- optional left section index;

- main content pane;

- more simultaneous open cards.

Same content.

Responsive presentation only.

44. No horizontal tables in field Full Mode

Important.

Tables may look good in our planning documents, but on tablets/phones they become annoying.

Prefer:

Label
value

or cards/lists.

Use tables only where the data genuinely requires comparison.

45. Keep technical IDs visible but subtle

Example:

8.9 Mechanical Penetrations

The ID helps cross-reference.

But don't make the interface look like an accounting system.

Activity title remains dominant.

46. Full Mode search within activity

I think Full should eventually allow:

Find on this activity

Search:

sleeve

and jump to all relevant Full sections.

Useful for long activities.

However, this is Build 6, not core 5C.

We reserve capability; don't spend time building it now.

47. Full Mode completion marker

Because this is a guide, not project software, don't place:

Mark inspection complete

inside Full MVP.

But it may have:

Review Quality Checkpoint

at the end.

That keeps the guide from masquerading as CRT's project record system.

48. Printing/export is not MVP priority

Full Mode could theoretically become printable someday.

Not now.

It adds little to field usefulness before August 31.

Keep it out.

49. Full Mode usability test

Before accepting any Full page, ask:

1.

Can I identify the objective immediately?

2.

Can I find governing-document sources quickly?

3.

Can I find the detailed inspection criteria without reading everything?

4.

Are pre-concealment requirements obvious?

5.

Are interfaces reachable?

6.

Are specialist boundaries obvious?

7.

Can I find evidence requirements quickly?

8.

Can I determine what to do if something is wrong?

9.

Can I determine closure criteria?

10.

Is anything duplicated unnecessarily?

If all ten pass, Full is doing its job.

50. Final Full Mode presentation model

Conceptually:

Activity Header

Quick | Full | Learn

Relationship Strip

Before | Gate | Interfaces | Testing | After | Closeout

Critical Flags

Pre-Concealment / High-Control / Specialist

Quality Objective ▼

Requirements ▶

Planning ▶

Document Control ▶

Material / Equipment ▶

Inspection / Testing ▼

Records / Evidence ▶

NC / Deficiency ▶

Corrective Action ▶

Verification ▶

Closure ▶

Communications ▶

Outputs ▶

Reporting / Analysis ▶

Quality Checkpoint ▶

[Section Jump]

This is the right balance between thoroughness and usability.

BUILD 5C — FROZEN ✓

We now have:

Accordion presentation ✓
Default-open sections ✓
Persistent activity context ✓
Relationship strip ✓
Grouped requirements ✓
Structured planning ✓
Standardized document control ✓
Material/equipment structure ✓
Before/During/After/Test inspection grouping ✓
Nested inspection subgroup rule ✓
Evidence organization ✓
Common issue + escalation structure ✓
Common corrective-control pattern ✓
Verification separation ✓
Closure criteria ✓
Communications ✓
Outputs ✓
Reporting/managerial insight ✓
Quality checkpoint ✓
Inline-link rules ✓
Conditional section hiding ✓
Gate/test/deficiency anti-duplication ✓
Specialist boundary card ✓
Invalidation notice ✓
Five specialized Full templates ✓
Tablet/PC behavior direction ✓
10-point usability test ✓

Next is 5D — Learn Mode Presentation: defining exactly how the Build 2 knowledge becomes an actual learning system—terminology, “how good work looks,” failure reasoning, practical scenarios, interfaces, and professional judgment—without turning Learn into a construction textbook.

Yes. 5D is Learn Mode Presentation.

This is where the guide stops being just a field checklist and becomes a professional-development system.

The objective is not to dump more text on you. It is to answer:

What is this work, why does it matter, what should good work look like, how does it fail, how do the interfaces behave, and how should I reason when I encounter it?

Build 2 already contains the raw ingredients for this: quality objectives, technical inspection logic, common deficiencies, corrective examples, verification, closure, reporting/analysis, and practical situations. For example, its mechanical-penetration module explains not only what to inspect but why a penetration belongs simultaneously to mechanical, structural, envelope/architectural, and fire/life-safety systems.

BUILD 5 — STEP 5D

LEARN MODE PRESENTATION

1. Learn Mode purpose

Learn should answer three progressively deeper questions:

Level 1 — Understand

What am I looking at?

Level 2 — Reason

Why are these checks important?

Level 3 — Apply

How do I recognize good work, diagnose bad work, and connect it to other systems?

That gives us the structure.

2. Learn should not duplicate Full

This rule is critical.

Full says:

Check opening size, sleeve, firestop, air/water seal, insulation continuity.

Learn says:

Why opening size matters, what a sleeve does, why an oversized opening may no longer match the approved firestop system, and how the same penetration affects several assemblies.

So:

Full = procedure.
Learn = understanding.

3. Learn Mode exact structure

I would freeze nine blocks:

- What Is This?

- Why It Matters

- Know the Terms

- How Good Work Looks

- Critical Checks Explained

- Common Failures

- Practical Examples

- Interfaces & Sequence

- Specialist / Authority Boundary

That is enough.

4. WHAT IS THIS?

This is the orientation block.

Maximum:

roughly 1–3 compact paragraphs.

It answers:

- what the activity/system is;

- where it normally appears;

- what function it performs;

- what QC is trying to control.

Example:

MECHANICAL PENETRATIONS

A mechanical penetration is an opening through another construction assembly that allows pipe, duct, or another mechanical service to pass. Quality control is not limited to verifying the service itself: the opening can affect the surrounding structure, fire resistance, air/water control, insulation, acoustics and finish.

That stays faithful to Build 2's explicit treatment of penetrations as multidisciplinary work.

5. WHY IT MATTERS

This should explain performance consequences, not repeat the objective.

Possible dimensions:

Structural

Could affect support/load path/opening integrity.

Water

Could leak.

Air

Could compromise enclosure continuity.

Fire

Could compromise a rated separation.

Thermal

Could create insulation discontinuity.

Operational

Could interfere with equipment access/function.

Durability

Could produce long-term deterioration.

Only applicable dimensions appear.

So Learn gradually trains you to ask:

“What performance function am I protecting?”

rather than just:

“What checklist item am I checking?”

6. KNOW THE TERMS

This is where Build 4 will eventually integrate strongly.

The card should look like:

KNOW THE TERMS

Sleeve
Short explanation.

Annular space
Short explanation.

Backer rod
Short explanation.

Firestop system
Short explanation.

Penetrating item / penetrant
Short explanation.

Later:

Sleeve — Manchon

Annular space — Espace annulaire

etc.

But Build 5D only freezes the presentation structure. Build 4 will establish the approved EN/FR terminology.

7. Terminology should be contextual

Do not build a dictionary inside every Learn page.

Show only terms that are:

- important to understanding the activity;

- likely to appear in plans/specs/site conversations;

- easy to misunderstand;

- necessary for inspection reasoning.

Usually around:

3–8 terms

Then users can jump to the full glossary later.

8. HOW GOOD WORK LOOKS

I consider this one of the most important Learn components.

Most inspection manuals over-emphasize defects.

But field recognition improves if you first know:

What should a properly executed condition look like?

This block should explain the expected final condition in plain technical language.

Example:

GOOD MECHANICAL PENETRATION

A good penetration has:

- a coordinated opening;

- no unauthorized structural damage;

- correct sleeve/configuration where required;

- properly supported service;

- restored fire rating where applicable;

- continuous air/water layer where applicable;

- restored insulation;

- finished closure appropriate to the assembly;

- traceable evidence where required.

Build 2 already frames the quality objective exactly around restoring those surrounding performance requirements.

9. “Good Work” should be visual in language

Use phrases such as:

“You should expect to see…”

“The completed condition should show…”

“At this stage, the assembly should already have…”

rather than abstract specification language.

This helps build visual recognition.

10. CRITICAL CHECKS EXPLAINED

This is where Quick and Learn connect.

Quick may say:

☐ Required depth

Learn can explain:

Why installed depth matters

The installed material depth forms part of the approved firestop configuration. A visually complete surface seal does not necessarily prove the required system configuration exists beneath the surface.

So Learn doesn't explain every checkbox.

Only the important or non-obvious ones.

11. Selection rule for explained checks

Explain a Quick/Full check when:

- the reasoning is not obvious;

- there is a common misconception;

- acceptance depends on configuration;

- interaction with another system matters;

- there is significant concealment risk;

- specialist boundaries matter.

Usually:

3–6 critical checks explained

per activity.

12. COMMON FAILURES

This expands Quick's Watch For.

Quick:

Oversized opening.

Learn:

Oversized opening

What you see
The hole is visibly larger than required around the service.

Why it matters
The installed geometry may no longer match the applicable penetration/firestop/detail requirements.

QC response
Do not assume extra sealant fixes the condition. Compare against the approved system/detail and obtain appropriate disposition where necessary.

Build 2 contains this exact type of firestop penetration scenario: when the opening falls outside the approved system range, the condition must be documented and controlled while an approved solution or authorized disposition is established.

13. Failure-card format

I recommend every important failure use:

CONDITION

What it looks like

Why it matters

What to verify next

That is enough.

We do not need:

symptom → root cause → mechanism → forensic analysis → dissertation.

Learn must remain usable.

14. COMMON FAILURES vs WATCH FOR

Freeze this distinction:

Quick — Watch For

“What should my eyes notice?”

Learn — Common Failures

“What does that condition mean and why should I care?”

Perfect.

15. PRACTICAL EXAMPLES

Build 2 already contains many good practical scenarios. These should become mini case studies.

The standard format should be:

SITUATION

What is happening?

OBSERVATION

What did QC notice?

QUALITY CONCERN

Why could it matter?

REASONING

What systems/requirements should be checked?

ACTION PATH

What is controlled, communicated, corrected, tested or escalated?

CLOSURE

What proves the issue is resolved?

This transforms Learn into something much more useful than definitions.

16. Example — roof drain

Build 2 gives a case where the finished roof membrane slopes to a low point away from the drain. Its workflow is essentially: observe/survey ponding geometry → compare against the taper plan → obtain authorized correction → rework insulation/membrane as required → retest drainage → document.

Learn could present:

SITUATION

Water ponds approximately 1.5 m away from the roof drain.

OBSERVATION

The drain itself may be properly connected, but it is not functioning as the effective low point.

THINK

This is not merely a plumbing issue. Check:

- roof slope/taper;

- insulation geometry;

- membrane;

- drain elevation.

ACTION PATH

Document → compare approved roof geometry → obtain authorized corrective direction → rework affected assembly → verify drainage → document closure.

That teaches systems thinking.

17. Example — low coating thickness

Build 2 gives another excellent example: deficient DFT readings lead to mapping the deficient area, confirming the requirement, preparing the surface where needed, applying additional coating, remeasuring, and documenting accepted results.

Learn should explain:

The lesson isn't just “add more paint.”

It's:

- verify the required system;

- identify affected area;

- ensure recoat preparation is compatible;

- restore required thickness;

- measure again;

- retain evidence.

That's quality reasoning.

18. Example — corrective work creates another defect

Build 2 explicitly warns that a correction can itself be deficient—for example repairing damaged waterproofing but failing to reinstate the protection board. The full system must be restored before the issue is ready for reinspection.

That is a fantastic Learn lesson:

Do not inspect only the repaired defect. Inspect the affected assembly.

That principle belongs in Learn and QC Think.

19. INTERFACES & SEQUENCE

This block explains the Build 3 graph in human terms.

Example:

MECHANICAL PENETRATION

Before
Structure/opening must be coordinated.

During
Mechanical service passes through assembly.

Also affected

- structural condition;

- air barrier;

- insulation;

- firestop;

- finish.

After
Assembly is closed.

That teaches the user to mentally see the construction sequence.

20. Interface presentation

Use:

CONNECTS TO

Structure
Why the opening matters.

Envelope
Why air/water continuity matters.

Fire/Life Safety
Why rated penetration control matters.

Architectural
Why final closure/finish matters.

Then links:

Open Structural Interface →

Open Firestopping →

This is where Learn explains what Quick only links.

21. Sequence presentation

Learn should show simple flows.

Example:

Opening → Sleeve → Service → Air/Water Restoration → Insulation → Firestop → Finish

or:

Subgrade → Granular → Reinforcement/Embeds → Pre-Pour → Concrete → Curing → Acceptance

Simple arrows are very effective for learning.

No complicated Gantt charts.

22. SPECIALIST / AUTHORITY BOUNDARY

This section is mandatory when Build 3 marks specialistInterface.

Format:

YOUR QC ROLE

What physical/conformity/documentation checks you can reasonably perform.

SPECIALIST / AUTHORIZED ROLE

What belongs to:

- engineer;

- electrician;

- lab;

- NDT;

- commissioning specialist;

- fire-alarm specialist;

- other authorized party.

ESCALATE WHEN

Specific conditions where you should not improvise.

Build 2 repeatedly preserves this boundary. For emergency lighting, for example, physical location/orientation/visibility can be checked by QC, while the functional electrical test remains with authorized electrical personnel.

23. Specialist boundary example — electrical

YOUR QC ROLE

- correct equipment;

- physical location;

- anchorage;

- clearance;

- identification;

- interfaces;

- required test records present.

AUTHORIZED ELECTRICAL ROLE

- specialized electrical acceptance;

- energization;

- electrical performance interpretation;

- specialist test acceptance.

ESCALATE

- test failure;

- conflicting technical requirements;

- grounding/bonding technical uncertainty;

- electrical design issue.

That prevents Learn from manufacturing false confidence.

24. Learn should integrate QC THINK

At the bottom:

QC THINK — APPLY IT

For the current activity, answer:

What governs this?

What becomes hidden?

What other system does it affect?

What evidence proves compliance?

What happens next?

Has anything changed since previous acceptance?

Then optionally:

Show Answers / Guidance

This could eventually become a self-study exercise.

Very useful.

25. Optional “Think First” behavior

For study mode, before showing a practical example's reasoning, we could show:

What would you check first?

Then tap:

Reveal reasoning

This encourages active learning.

But I would classify this as nice-to-have, not MVP-critical.

26. Learn should include managerial insight selectively

Because the long-term objective of the framework is stronger quality thinking, some activities can include:

QUALITY MANAGEMENT INSIGHT

Example:

Repeated missed embeds across several pours may indicate a coordination/process failure rather than independent installer mistakes.

Build 2 itself makes this point by recommending tracking missed embeds because they can indicate coordination breakdown.

This helps develop the transition from:

“I found a defect.”

to:

“Why does this defect keep occurring, and what process is failing?”

27. Managerial insight should not appear everywhere

Only where Build 2 Reporting/Analysis genuinely contains useful system-level learning.

Examples:

- recurring failed tests;

- repeated coordination conflicts;

- repeated missed embeds;

- documentation bottlenecks;

- supplier/material problems;

- repeated access problems.

Otherwise omit the card.

28. Learn Mode terminology hover/tap

On PC:

Hover term → short definition.

On tablet:

Tap term → short definition.

Example:

annular space ⓘ

This can eventually reduce the need for a huge visible glossary section.

But the dedicated Know the Terms card still remains for the most important vocabulary.

29. Learn Mode does not need checkboxes

Important visual distinction.

Quick/Full are operational.

Learn is explanatory.

So prefer:

- short paragraphs;

- diagrams/flows;

- callout cards;

- examples;

- term definitions.

Not:

☐ Learn this
☐ Understand that

That would make Learn feel like another inspection form.

30. Learn Mode length control

We need strong limits.

For a normal activity:

What Is This

~100–200 words maximum.

Why It Matters

~3–6 short points.

Terms

~3–8.

Good Work

~4–8 characteristics.

Critical Checks Explained

~3–6.

Common Failures

~3–6.

Examples

Usually 1–3.

Interfaces

Only important ones.

This prevents Learn from rebuilding Build 2 verbatim.

31. High-control activities can go deeper

For nodes such as:

- 2.4 Pre-Pour;

- 4.2 Air Barrier;

- 5.4 Roofing Membrane;

- 8.9 Mechanical Penetrations;

- 9.9 Electrical Penetrations;

- 10.3 Firestopping;

- 13.9 Failed-Test Management;

Learn can be somewhat richer.

But still layered/collapsible.

32. Learn Mode card behavior

Recommended default:

WHAT IS THIS?                   ▼

WHY IT MATTERS                 ▼

KNOW THE TERMS                 ▶

HOW GOOD WORK LOOKS            ▼

CRITICAL CHECKS EXPLAINED      ▶

COMMON FAILURES                ▶

PRACTICAL EXAMPLES             ▶

INTERFACES & SEQUENCE          ▶

SPECIALIST BOUNDARY            ▶

QUALITY MANAGEMENT INSIGHT     ▶

Open by default:

- What Is This

- Why It Matters

- How Good Work Looks

Everything else expandable.

That gives immediate understanding.

33. Learn Mode example — Firestopping

Conceptually:

WHAT IS THIS?

A firestop system restores the required fire-resistance performance where services penetrate a rated assembly.

WHY IT MATTERS

A small unprotected opening can undermine the function of an otherwise acceptable rated wall/floor.

KNOW THE TERMS

- rated assembly;

- penetrant;

- annular space;

- backing;

- installed depth;

- listed system.

GOOD WORK LOOKS LIKE

- correct assembly identified;

- opening/service configuration matches applicable approved system;

- correct components;

- complete installation;

- documented before concealment.

CRITICAL CHECKS EXPLAINED

- opening geometry;

- annular space;

- backing;

- installed depth.

COMMON FAILURES

- oversized opening;

- wrong system;

- missing backing;

- inadequate depth;

- later damage.

PRACTICAL EXAMPLE

Pipe penetration falls outside approved system range → control → obtain suitable system/disposition → install → inspect → document → close.

That example is directly supported by Build 2.

INTERFACES

Rated wall ↔ mechanical/electrical service ↔ firestop ↔ closure.

SPECIALIST BOUNDARY

Use approved firestop systems/project requirements; escalate conditions outside approved configuration.

That's a genuinely useful learning page.

34. Learn Mode example — Acceptance Evidence

This proves Learn also works for non-physical Section 13/14 activities.

Build 2 defines acceptance evidence as a complete, traceable package proving that a system was inspected, tested, corrected where necessary, accepted by the appropriate authority, and is ready for turnover.

Learn can explain:

WHAT IS THIS?

The evidence package tells the complete quality story.

WHY IT MATTERS

A system can physically work and still be incomplete from a contractual/QC perspective if required evidence is absent.

GOOD WORK LOOKS LIKE

The chain can be reconstructed:

Requirement → Inspection → Test → Failure → Correction → Retest → Acceptance → Turnover evidence

Build 2 explicitly asks whether the entire chain can be reconstructed.

COMMON FAILURES

- missing test report;

- unsigned form;

- unresolved failure;

- equipment-ID mismatch;

- missing startup record;

- open deficiency;

- inconsistent as-built.

This teaches why documentation is part of quality, not bureaucracy after quality.

35. Learn Mode should support cross-learning

At the bottom:

CONTINUE LEARNING

For Firestop:

- Fire-Rated Assemblies

- Mechanical Penetrations

- Electrical Penetrations

- Universal Penetrations

- Wall Closure

These should come from Build 3 relationships.

No manually curated duplicate list unless ranking is needed.

36. “Continue Learning” ranking

Prioritize:

- direct prerequisite;

- critical interface;

- immediate downstream work;

- universal workflow;

- testing/commissioning.

Maximum:

3–5 suggestions.

37. Learn Mode and bilingual content

Once Build 4 is done:

The language toggle should affect:

- titles;

- terminology;

- short explanations;

- technical labels.

For someone learning French terminology, EN/FR mode could be particularly useful in Learn:

Backer rod / Fond de joint

Sleeve / Manchon

Reinforcement / Armatures

But Build 4 will determine the actual approved wording.

38. Learn should not silently introduce outside technical criteria

Same universal rule:

Learn can explain concepts.

It should not invent:

- required fire rating;

- concrete cover;

- test pressure;

- spacing;

- torque;

- membrane thickness;

- slope;

- code requirement.

Those remain:

as specified / as approved / according to governing project documents

unless the source itself establishes a universal conceptual statement.

39. Source fidelity rule

Because Learn transforms Build 2 more aggressively than Quick/Full, we need this rule:

Every Learn explanation must be traceable either to Build 2 content or clearly identified later as an approved supplemental educational explanation.

For the initial MVP, I strongly recommend we keep Learn predominantly grounded in Build 2.

Later, if we deliberately expand technical education, we can clearly add researched reference material.

That avoids accidental drift from our original QC framework.

40. Learn content derivation

The source fields map neatly:

| Build 2 field | Learn output |
| --- | --- |
| Quality Objective | What Is This / Why It Matters |
| Requirements | Terminology/context |
| Planning | Sequence |
| Inspection | How Good Work Looks / Critical Checks |
| Common Deficiencies | Common Failures |
| Corrective Action | Practical Examples |
| Verification | What acceptable correction looks like |
| Closure | Good final condition |
| Reporting/Analysis | Quality Management Insight |
| Build 3 Relationships | Interfaces & Sequence |
| Specialist Boundary | Authority Boundary |
| Invalidation Rules | “What can undo acceptance?” |

So Learn can be built systematically.

41. “What can undo acceptance?” mini-card

Because Step 6 was important, I would include this for selected activities.

WHAT CAN CHANGE THIS LATER?

Example Firestop:

- new cable/conduit;

- penetration enlargement;

- damage;

- removal/rework.

Example roof:

- new penetration;

- membrane puncture;

- relocated equipment;

- curb modification.

This develops temporal awareness:

Quality isn't only what I inspect now; later trades can alter it.

Not every page needs this.

42. Learn Mode validation test

Before approving a Learn page:

1.

Can a professional unfamiliar with the activity understand what it is?

2.

Does it explain why QC cares?

3.

Does it describe good work, not only defects?

4.

Are important terms explained?

5.

Does it explain the reasoning behind key checks?

6.

Are common failures practical and recognizable?

7.

Is at least one practical scenario useful where Build 2 supports one?

8.

Are interfaces explained?

9.

Is specialist authority preserved?

10.

Did we avoid turning it into Full Mode with paragraphs?

11.

Did we avoid inventing project-specific requirements?

12.

Can the user finish the page with a better mental model than before?

If yes, Learn succeeds.

43. Canonical Learn object

Conceptually:

LearnView {

activityId

whatIsThis

whyItMatters[]

terminology[]

goodWorkLooksLike[]

criticalChecksExplained[]

commonFailures[]

practicalExamples[]

interfaceExplanations[]

sequenceFlow[]

specialistBoundary

invalidationInsight

qualityManagementInsight

continueLearning[]

}

Again:

deterministic structured content, not runtime AI generation.

44. Quick / Full / Learn now form a coherent ladder

Take one activity:

QUICK

“Check membrane continuity, openings, penetrations, evidence.”

FULL

“Here are all requirements, planning, documents, materials, inspection steps, records, issues, verification and closure criteria.”

LEARN

“Here is why continuity matters, what good installation looks like, why transitions fail, how penetrations interact with the system, and how to reason when you find a defect.”

That's exactly the layered system we wanted.

BUILD 5D — FROZEN ✓

We have now established:

Learn purpose ✓
Nine Learn blocks ✓
Understand → Reason → Apply model ✓
Learn ≠ Full distinction ✓
Terminology structure ✓
How Good Work Looks ✓
Critical-check explanation rules ✓
Common Failure structure ✓
Practical case-study structure ✓
Interface/sequence explanation ✓
Specialist authority boundary ✓
QC Think integration ✓
Managerial insight ✓
Invalidation learning ✓
Length limits ✓
High-control depth rule ✓
Learn accordion behavior ✓
Continue Learning relationships ✓
Build 2 → Learn derivation rules ✓
Source-fidelity rule ✓
12-point Learn validation test ✓

5E is next: Activity Mode. That's where we define the “What are you doing?” composite workflows—Concrete Pour, Wall Closure, Roof Inspection, Pressure Test, Equipment Installation, Deficiency Walk, Commissioning, etc.—and make them intelligently pull from multiple Build 2/Build 3 modules without duplicating their content.

Yes. 5E is Activity Mode — the “What are you doing right now?” layer.

This is one of the most useful parts of the whole system because field work often starts from an event, not from a discipline.

You may not think:

“I need Section 2 Substructure.”

You think:

“We’re pouring concrete in 45 minutes.”

or:

“They’re closing this wall.”

or:

“We’re testing this piping system.”

So Activity Mode should convert those real field situations into a temporary composite workflow assembled from the canonical Build 2 activities, Build 3 relationships, and gates.

Critically:

Activity Mode contains no independent technical content.

It orchestrates existing content.

BUILD 5 — STEP 5E

ACTIVITY MODE / COMPOSITE FIELD WORKFLOWS

1. Core purpose

Activity Mode answers:

What is happening on site, and what QC systems need to come together for this event to proceed correctly?

It sits above individual activity pages.

So we now have two ways into the same knowledge base:

Browse by construction system

Mechanical → HVAC Ductwork

Browse by field event

What are you doing? → Ceiling Closure

Both eventually open the same canonical activities.

2. Activity Mode should not create duplicate pages

This is the most important rule.

For example:

“Concrete Pour”

does not get its own separate reinforcement checklist, embed checklist, concrete checklist, testing procedure, and deficiency workflow.

Instead it composes:

- 2.1 Foundation Formwork

- 2.2 Foundation Reinforcement

- 2.3 Embedded Items

- 2.4 Pre-Pour Quality Gate

- 2.5 Foundation Concrete

plus applicable:

- 9.1 Embedded Conduits

- 9.6 Grounding/Bonding

- 11.8 Embedded Items

- concrete testing.

Build 2 explicitly treats the pre-pour state this way: formwork, reinforcement, embeds, documents, site conditions and testing arrangements converge before concrete proceeds.

3. Composite workflow structure

Each field workflow should have:

WHAT IS HAPPENING?

Brief event definition.

BEFORE

What must already be ready.

KEY QC STREAMS

The canonical activities involved.

GATE

The controlling gate, if any.

DURING

What activities matter while the event is occurring.

AFTER

Immediate downstream checks.

EVIDENCE

What evidence is particularly important.

ISSUES

What common failure path applies.

RELATED

Links to the underlying Full/Quick/Learn pages.

That's enough.

4. Activity Mode should be shorter than Full

It is coordination-oriented, not content-heavy.

Think:

mission dashboard

rather than:

inspection manual

For a concrete pour, you should be able to glance and see:

Formwork ✓
Rebar ✓
Embeds ✓
MEP cast-ins ✓
Gate ✓
Testing ready ✓
Placement / testing / curing next

Then tap the specific item that needs depth.

5. Canonical Activity Mode workflow object

Conceptually:

CompositeWorkflow {

id

title

purpose

prerequisiteActivityIds[]

gateIds[]

activeActivityIds[]

downstreamActivityIds[]

evidencePriorities[]

commonRiskLinks[]

relatedWorkflowIds[]

}

No duplicated technical requirements.

6. Initial workflow set

I would start with 12 high-value field workflows.

Not 40.

These cover most situations where Activity Mode adds real value.

Structural / civil

- Concrete Pour

- Underground Service Before Backfill

- Structural Steel Erection

Envelope / architectural

- Wall Closure

- Ceiling Closure

- Roof Inspection / Roof Closure

- Roof Penetration

MEP

- Equipment Installation

- Pressure / Leak Test

- Equipment Start-Up / Commissioning Readiness

Fire / completion

- Firestop Inspection

- Deficiency Walk

Later we can add more only if field use shows value.

7. WORKFLOW 1 — CONCRETE POUR

Purpose

Bring all pre-pour prerequisites together before concrete permanently conceals the work.

BEFORE

Structure

- 2.1 Foundation Formwork

- 2.2 Foundation Reinforcement

- 2.3 Embedded Items

MEP / Interface

- 9.1 Embedded Conduits where applicable

- 9.6 Grounding/Bonding where applicable

- mechanical sleeves/openings

- 11.8 Embedded Items

GATE

G-STR-01 Structural Pre-Pour Gate

and applicable Build 2 pre-pour quality gate logic.

DURING

- 2.5 Foundation Concrete / 3.1 Cast-in-Place Concrete

- placement

- consolidation

- sampling/testing

- finishing

AFTER

- curing

- surface condition

- dimensional check

- 2.6 Concrete Acceptance where applicable

EVIDENCE

- pre-pour photos

- rebar/embeds condition

- gate/release evidence

- test/sampling records

KEY FAILURE PATH

Missing/moved embed, reinforcement issue, test-readiness issue:

Stop affected progression → correct → reinspect → release.

8. WORKFLOW 2 — UNDERGROUND SERVICE BEFORE BACKFILL

This should be very strong because once buried, evidence becomes expensive or impossible to recreate.

BEFORE

- 12.2 Utility Trenches

- 12.1 Underground Services

- applicable mechanical/electrical service

- survey/layout

VERIFY

- route

- elevation

- slope where applicable

- bedding

- joints

- service ID

- structures/connections

- protection

TEST

Required service test.

GATE

G-EXT-01 Pre-Backfill Gate

EVIDENCE

- location

- elevation

- photos

- test

- as-built survey

AFTER

- backfill

- compaction

Build 2's external-works logic explicitly requires inspection, testing, pre-concealment evidence and as-built information before burial.

9. WORKFLOW 3 — STRUCTURAL STEEL ERECTION

BEFORE

- 3.2 Structural Steel — Materials

- accepted supports/anchors

- survey/layout

- erection/shop drawings

ACTIVE QC

- 3.3 Steel Erection

- member ID

- location/orientation

- alignment/elevation/plumbness

- temporary bracing

CONNECTIONS

- 3.4 Bolted Connections

- 3.5 Welding

AFTER

- 3.6 Steel Protection

- acceptance/survey as applicable

INTERFACES

- envelope

- MEP supports/openings

- equipment support

- architectural closure

TESTING

- bolt inspection/testing

- weld inspection/NDT where required

10. WORKFLOW 4 — WALL CLOSURE

This should be one of the most useful workflows in the entire application.

BEFORE

Wall

- 6.2 Metal Stud Framing or 6.1 Masonry

- backing/reinforcement

- 6.3 Interior Insulation

MEP

- mechanical rough-in

- electrical rough-in

- boxes/sleeves

- required testing

Fire / acoustic

- 6.5 Fire-Rated Assemblies

- 10.3 Firestopping

- acoustic requirements

Access

- 6.9 Access Panels / 11.10 Access & Maintainability

GATES

- G-MEP-01

- G-INT-01

EVIDENCE

- concealed-condition photos

- firestop evidence

- rough-in/test evidence

- wall type/location

AFTER

- 6.4 Gypsum Board

- wall finishes

Build 2 specifically makes wall/ceiling closure dependent on framing, MEP, tests, firestop, access and unresolved conditions.

11. WORKFLOW 5 — CEILING CLOSURE

Similar to wall closure, but the interface mix changes.

BEFORE

- ceiling support/grid conditions

- ductwork

- piping

- conduits/trays

- lighting

- fire alarm

- sprinklers/fire protection

- dampers

- access panels

- insulation where applicable

- required tests

- open deficiencies

GATES

G-MEP-01

G-INT-01

DON'T MISS

- damper access

- valve access

- junction/equipment access

- unclosed penetrations

- required above-ceiling testing

AFTER

- ceiling panels/tiles

- final device alignment

- deficiencies

12. WORKFLOW 6 — ROOF INSPECTION / ROOF CLOSURE

This one should expose the roof progression gates.

BEFORE

- 5.1 Roof Deck/Substrate

PROGRESSION

G-ROOF-01A
→ 5.2 Air/Vapour Layer

G-ROOF-01B
→ 5.3 Insulation

G-ROOF-01C
→ 5.4 Roofing Membrane

G-ROOF-01D
→ 5.5 Flashings
→ 5.6 Penetrations
→ 5.7 Curbs
→ 5.8 Drains

FINAL

G-ROOF-01E
→ 5.9 Roofing Final Inspection/Testing

WATCH FOR

- wet substrate/insulation

- punctures

- poor seams

- incomplete terminations

- penetration damage

- ponding/drain issues

Build 2 explicitly defines these staged roof gates.

13. WORKFLOW 7 — ROOF PENETRATION

This deserves its own composite workflow because it is highly multidisciplinary.

SYSTEMS

- 5.6 Roof Penetrations

- 8.9 Mechanical Penetrations or 9.9 Electrical Penetrations

- 11.7 Universal Penetrations

- 5.4 Roofing Membrane

- 5.3 Insulation

- 5.2 Air/Vapour Layer

CHECK

- opening

- sleeve/curb

- service

- structure

- membrane

- insulation

- flashing

- air/vapour continuity

- firestop if applicable

EVIDENCE

- location

- service

- before/after photos

- roof integration

- system detail

INVALIDATION

New roof penetrations can reopen previously accepted roof conditions.

14. WORKFLOW 8 — EQUIPMENT INSTALLATION

BEFORE

- approved equipment

- 11.9 Equipment Base / Housekeeping Pad

- support/curb

- space/access

- service coordination

INSTALL

- 8.6 HVAC Equipment or 9.4 Electrical Equipment

- orientation

- anchorage

- vibration isolation where required

- clearances

- condition

INTERFACES

- piping

- ductwork

- drain

- electrical

- controls

- 11.10 Access & Maintainability

AFTER

- identification

- prerequisite testing

- G-MEP-02 readiness

- start-up/commissioning

15. WORKFLOW 9 — PRESSURE / LEAK TEST

This should be generic, with the specific system selected afterward.

SELECT SYSTEM

- Domestic Water

- Sanitary/Drainage where applicable

- Storm

- Process Piping

- Other mechanical system

BEFORE

- correct system/test boundary

- installation sufficiently complete

- current test requirement/procedure

- instruments/calibration

- personnel/witness

- G-TST-01

TEST

- configuration

- test condition

- reading

- leakage/behavior

- result

IF PASS

→ acceptance evidence / downstream work

IF FAIL

→ 13.9 Failed-Test Management
→ correction
→ 13.10 Retest

Build 2 explicitly preserves this fail–correct–retest–accept chain.

16. WORKFLOW 10 — EQUIPMENT START-UP / COMMISSIONING READINESS

BEFORE

Physical

- equipment installed

- anchors/support

- clearances

- guards/covers

Mechanical

- piping

- ductwork

- drain

Electrical

- supply/interface

- grounding/bonding where required

Controls

- sensors

- actuators

- BAS interfaces

Documentation

- manufacturer procedure

- identification

- required test results

GATE

G-MEP-02 Equipment Commissioning Readiness

NEXT

- 13.7 Equipment Start-Up

- 13.6 Functional Testing

- balancing where applicable

- acceptance evidence

Build 2's AHU example already follows installation → interfaces → start-up → functional testing → balancing → acceptance evidence.

17. WORKFLOW 11 — FIRESTOP INSPECTION

Although Firestop already has its own activity page, Activity Mode here is useful because users may start from:

“I'm doing the firestop walk.”

BEFORE

- rated assembly identified

- penetration/service identified

- approved firestop system

- opening/configuration understood

INSPECT

→ 10.3 Firestopping

RELATED

- 8.9 Mechanical Penetrations

- 9.9 Electrical Penetrations

- 10.1/10.2 Rated Assemblies

- 11.7 Universal Penetrations

PRE-CONCEALMENT

- photo

- system ID/reference

- location

- label where applicable

AFTER

- G-INT-01 / wall-ceiling closure

- life-safety readiness where applicable

INVALIDATION

Reinspect if penetration is later modified.

Build 2 explicitly requires reopened/reinspected firestop when a controlled penetration is modified later.

18. WORKFLOW 12 — DEFICIENCY WALK

BEFORE

- area/system sufficiently complete

- previous deficiency list reviewed

- applicable current documents available

REVIEW

Physical

- incomplete work

- damage

- workmanship

- operation

- access

- finishes

Interfaces

- architectural/MEP

- fire/life safety

- equipment

- penetrations

Existing corrections

- verify previously reported items

RECORD

- exact location

- clear description

- photo

- classification

- responsible trade

NEXT

- 14.3 Classification

- 14.4 Assignment

- 14.5 Corrective Work

- 14.6 Reinspection

- 14.7 Closure

Build 2 explicitly separates reported correction from verified closure.

19. Composite workflow views

Each Activity Mode workflow should itself have only two depth levels, not Quick/Full/Learn all over again.

I recommend:

FIELD

Default mission view.

DETAILS

Shows all constituent activities and relationships.

Why?

Because if we put Quick/Full/Learn inside Composite Workflow and then Quick/Full/Learn inside every linked activity, the UX gets messy.

Instead:

Activity Mode = coordination shell.

Tap a component:

Firestopping → Quick / Full / Learn

Much cleaner.

20. Field view layout

Example:

WALL CLOSURE

READY WHEN

☐ Framing/backing complete
☐ MEP rough-ins complete
☐ Required tests complete
☐ Insulation complete
☐ Firestop complete
☐ Access maintained
☐ Evidence captured
☐ Blocking issues resolved

GATES

G-MEP-01
G-INT-01

QC STREAMS

Framing →
Mechanical →
Electrical →
Firestop →
Access →

NEXT

→ Gypsum / ceiling closure

DON'T MISS

Do not conceal unresolved MEP/fire/access conditions.

That's enough.

21. Details view

Tap:

Details

Then show:

Prerequisites

Constituent Activities

Interfaces

Gate logic

Evidence priorities

Downstream activities

Issue paths

Still no duplicate technical criteria.

22. Workflow composition types

I think we should classify workflows into four types.

closureWorkflow

Example:

- Wall Closure

- Ceiling Closure

- Backfill

executionWorkflow

Example:

- Concrete Pour

- Steel Erection

- Equipment Installation

testingWorkflow

Example:

- Pressure Test

- Commissioning

reviewWorkflow

Example:

- Roof Inspection

- Firestop Walk

- Deficiency Walk

This helps determine presentation emphasis.

23. Closure workflow emphasis

Show:

- prerequisites;

- concealment;

- evidence;

- gates;

- blocking issues.

Less emphasis on detailed execution.

24. Execution workflow emphasis

Show:

- prerequisites;

- active QC streams;

- during-work inspection;

- testing;

- immediate post-work checks.

25. Testing workflow emphasis

Show:

- readiness;

- procedure;

- instruments;

- test boundaries;

- personnel;

- result;

- failure/retest route.

26. Review workflow emphasis

Show:

- scope;

- inspection coverage;

- known risk areas;

- evidence;

- issue routing.

27. Activity Mode should reuse gate checklists

No manual duplication.

Example:

Wall Closure calls:

G-INT-01

and displays its key conditions.

Concrete Pour calls:

G-STR-01.

Underground Backfill calls:

G-EXT-01.

Equipment Start-Up calls:

G-MEP-02.

The gate remains the single source of truth.

28. Activity Mode should reuse relationship graph

Suppose later Build 3 changes the relation:

Damper → Access Panel

The Ceiling Closure workflow should pick that relationship up.

We should avoid manually hardcoding every interface inside composite workflows where the graph can derive it.

However—

some workflows need curated inclusion rules, because showing every graph edge would create clutter.

So the workflow object should identify:

- core constituent activities;

- relevant gate;

- relationship expansion depth.

29. Relationship expansion depth

For Activity Mode:

Depth 1 only by default.

Example:

Wall Closure contains Mechanical Rough-In.

Mechanical Rough-In relates to Equipment.

Equipment relates to Controls.

We do not automatically bring Controls into Wall Closure unless it actually affects the closure.

No runaway graph expansion.

30. Core vs conditional constituents

Composite workflow entries need:

core

Always relevant to the generic workflow.

conditional

Relevant only when that condition exists.

Example Wall Closure:

Core

- framing

- MEP rough-in

- insulation where wall assembly includes it

- evidence

Conditional

- firestop if rated penetrations

- acoustic insulation

- access panels

- fire/smoke damper if wall condition includes one.

UI:

Check if applicable

31. Workflow conditions

Use the controlled Build 3 vocabulary:

- whereRated

- whereExterior

- whereConcealed

- whereEquipmentPresent

- whereTestingRequired

- whereFireSeparation

- whereSpecified

No random free-text logic.

32. Activity Mode and specialist boundaries

Testing/commissioning workflows must prominently preserve:

QC coordination does not equal specialist technical acceptance.

For example Pressure Test:

Verify applicable procedure, witness and acceptance responsibilities.

Electrical testing:

Coordinate with authorized electrical personnel.

Integrated life safety:

Required specialist/authority participation applies.

33. Workflow evidence priorities

Activity Mode should extract only event-critical evidence.

Examples:

Concrete Pour

- pre-pour evidence

- test/sample record

- pour location

Wall Closure

- concealed-condition photos

- firestop

- rough-in/test evidence

Underground Before Backfill

- survey

- photo

- test

- service ID

Deficiency Walk

- location

- description

- photo

No giant records list.

34. “Blocking issue?” shortcut

Every composite workflow should have:

⚠ Something wrong?

Tap:

Physical deficiency

→ Section 14

Formal non-conformance

→ NC workflow

Failed test

→ 13.9

Technical uncertainty

→ applicable specialist/Responsable qualité guidance

That is highly useful in field conditions.

35. “Related activities” should show completion order

For composite workflows, relationships should be presented as a sequence, not just a list.

Example Concrete Pour:

Formwork
↓
Reinforcement
↓
Embeds / MEP Cast-ins
↓
Pre-Pour Gate
↓
Concrete Placement
↓
Testing / Curing
↓
Acceptance

That is much easier to understand.

36. Parallel activities

Not everything is sequential.

Wall Closure:

Mechanical Rough-In

/

Framing

\

Electrical Rough-In

Firestop / Insulation / Access

↓

Pre-Concealment Gate

↓

Closure

We can render this later visually.

For MVP, grouped lists are enough.

37. Workflow stage model

Each composite workflow can use:

PREPARE

VERIFY

EXECUTE / OBSERVE

TEST

RELEASE / CLOSE

Not every workflow needs all five.

This gives consistent language.

Example Concrete Pour:

Prepare
documents/testing

Verify
form/rebar/embeds

Release
pre-pour gate

Observe
placement

Test
concrete testing

Close
post-pour evidence

38. Activity Mode home screen

I would design the landing page eventually as:

WHAT ARE YOU DOING?

Before Pour
Before Closing Wall
Before Closing Ceiling
Before Backfill
Roof Work
Equipment Installation
Testing
Firestop Walk
Deficiency Walk
Commissioning

Then:

View All Workflows

This is better than immediately showing 12 similarly sized buttons.

39. Context categories

Group workflows visually:

BUILD

Concrete Pour
Steel Erection
Equipment Installation

CONCEAL

Wall Closure
Ceiling Closure
Before Backfill
Roof Layer

TEST

Pressure Test
Equipment Start-Up
Commissioning

REVIEW

Roof Inspection
Firestop Walk
Deficiency Walk

Simple.

40. Search should also find workflows

Later Build 6:

Search:

pour

returns:

Workflow: Concrete Pour

and:

Activity: Foundation Concrete

Search:

close wall

returns:

Workflow: Wall Closure

This will make navigation very fast.

41. Activity Mode should not become job scheduling

Important boundary.

It does not track:

- planned dates;

- subcontractor schedule;

- daily lookahead;

- assignments;

- project progress percentages.

That would move us toward project-management/QMS software.

Activity Mode merely says:

“For this field event, here are the QC streams you should bring together.”

42. No live checklists required in MVP

Checkbox appearance can be used as visual inspection guidance.

We do not need to persist:

checked at 9:22 AM by Emmanuel

because that would create project-record behavior.

Later, if we ever intentionally build a personal temporary checklist feature, we can discuss it separately.

For now:

reference guide first.

43. Activity Mode does not override project sequence

Very important.

The workflows show a logical universal QC sequence.

Actual sequencing may differ.

Subtle notice:

Confirm actual project sequence and PIE/PRIE requirements.

No assumption that every contractor follows the exact same construction sequence.

44. Activity workflow Learn connection

Activity Mode itself doesn't need Learn.

But at the bottom:

UNDERSTAND THIS WORKFLOW

could point to the most relevant Learn pages.

Example Wall Closure:

- Fire-Rated Assemblies

- Firestopping

- MEP Penetrations

- Access & Maintainability

That's enough.

45. Activity Mode cross-link behavior

When you tap:

Firestop

from Wall Closure,

open:

10.3 Firestopping → Quick

and Back returns to:

Wall Closure

Canonical breadcrumb remains:

Fire & Life Safety → Firestopping.

This uses the Build 3 navigation-history rules.

46. “Why am I seeing this?” helper

Conditional item:

Firestop — if wall is rated.

Tap ⓘ:

Included because penetrations through rated assemblies require the fire/life-safety interface to be controlled before closure.

Useful, but optional for MVP.

47. Composite workflow validation test

Before approving any workflow:

1.

Does it represent a real field event?

2.

Does Activity Mode genuinely save navigation time?

3.

Are the constituent activities already canonical?

4.

Did we avoid copying their technical content?

5.

Are critical gates included?

6.

Are pre-concealment risks included?

7.

Are critical interfaces included?

8.

Are testing requirements linked?

9.

Is evidence prioritized?

10.

Is downstream work obvious?

11.

Are conditional items clearly qualified?

12.

Are specialist boundaries preserved?

13.

Can the workflow be understood in under ~30 seconds?

If not, simplify it.

48. Workflow count rule

I would freeze:

MVP target: 10–15 workflows maximum.

We currently have 12.

Excellent.

Don't create a workflow for:

“Install one access panel”

when the regular activity page already handles that perfectly.

Activity Mode should be reserved for events involving multiple modules, a major gate, or complex sequence.

49. Candidate future workflows

Not MVP, but possible later if field use justifies them:

- Foundation Waterproofing Before Backfill

- Window Installation / Envelope Opening

- Fire-Rated Door Acceptance

- Above-Ceiling Inspection

- Exterior Wall Closure

- Asphalt Placement

- Equipment Room Final Acceptance

- Life-Safety Integrated Test

- Final Turnover Review

We don't build them now unless needed.

50. Canonical workflow dataset

For development:

CompositeWorkflow {

id

titleEn

titleFr

type

purpose

stages[] {

stageId

label

coreActivityIds[]

conditionalActivityIds[]

}

gateIds[]

evidencePriorities[]

issueRoutes[]

primaryNextActivityIds[]

relationshipExpansionDepth

specialistBoundary

projectSequenceNotice

}

Again, Build 4 fills French later.

51. Activity workflow IDs

Use a separate namespace.

I recommend:

WF-

Examples:

WF-CON-01 Concrete Pour
WF-EXT-01 Underground Before Backfill
WF-STR-01 Structural Steel Erection
WF-INT-01 Wall Closure
WF-INT-02 Ceiling Closure
WF-ROOF-01 Roof Inspection
WF-ROOF-02 Roof Penetration
WF-MEP-01 Equipment Installation
WF-TST-01 Pressure / Leak Test
WF-COM-01 Equipment Start-Up
WF-FIRE-01 Firestop Inspection
WF-CLS-01 Deficiency Walk

This ensures nobody confuses workflows with Build 2 activity IDs or gate IDs.

52. Final relationship

We now have three separate object families:

Activity

8.9 Mechanical Penetrations

Gate

G-INT-01 Wall/Ceiling Pre-Concealment

Composite Workflow

WF-INT-01 Wall Closure

That's excellent architecture.

They do completely different jobs.

BUILD 5E — FROZEN ✓

We now have:

Activity Mode purpose ✓
No duplicate technical content ✓
Composite workflow object ✓
12 MVP workflows ✓
Concrete Pour ✓
Underground Before Backfill ✓
Structural Steel Erection ✓
Wall Closure ✓
Ceiling Closure ✓
Roof Inspection ✓
Roof Penetration ✓
Equipment Installation ✓
Pressure/Leak Test ✓
Equipment Start-Up ✓
Firestop Inspection ✓
Deficiency Walk ✓
Field vs Details view ✓
Workflow type classification ✓
Gate reuse ✓
Graph reuse ✓
Core vs conditional activities ✓
Depth-1 relationship expansion ✓
Evidence-priority rules ✓
Issue shortcut ✓
Sequence model ✓
Activity Mode homepage concept ✓
Project/QMS boundary preserved ✓
13-point workflow validation test ✓
WF- namespace ✓*

The next step is 5F — Pre-Concealment Mode, where we formalize the dedicated “Before Closing / Covering / Pouring / Backfilling” shortcut using the Build 3 gate architecture. That should be even leaner than Activity Mode because its entire purpose is essentially:

“Before this disappears, what must I verify?”

Yes. 5F is Pre-Concealment Mode — and this one should be extremely lean.

Its entire purpose is:

“This work is about to disappear. What must I verify before I lose visual access to it?”

Unlike Activity Mode, this is not asking what event is happening generally. It is specifically triggered by an irreversibility point: concrete placement, wall closure, ceiling closure, roof covering, backfill, enclosure, insulation, fire-rated closure, etc.

Build 2 repeatedly treats these moments as critical quality gates: pre-pour conditions must be accepted before concrete, concealed wall/ceiling conditions must be reviewed before closure, underground services must be inspected/tested/evidenced before backfill, and roofing is deliberately inspected layer-by-layer before later layers obscure the previous condition.

BUILD 5 — STEP 5F

PRE-CONCEALMENT MODE

1. Core purpose

Pre-Concealment Mode answers only four questions:

What is about to become hidden?

What must be inspected before that happens?

What evidence should exist before access is lost?

Is there anything unresolved that should prevent concealment?

That is it.

No explanations.

No detailed Learn content.

No giant Full workflow.

2. Homepage shortcut

This should be one of the biggest shortcuts in the interface:

⚠ BEFORE CLOSING / COVERING

Then choose:

Before Concrete Pour

Before Wall Closure

Before Ceiling Closure

Before Roof Layer

Before Backfill

Before Closing a Fire-Rated Assembly

Before Concealing MEP

Those seven cover the major universal concealment conditions we established.

3. Why this is separate from Activity Mode

There is overlap, but the mental trigger is different.

Activity Mode

“We are pouring concrete.”

Gives:

- preparation;

- active work;

- testing;

- after-work controls.

Pre-Concealment

“Concrete truck is coming and all this rebar/conduit is about to disappear.”

Gives:

- only what must be checked before concealment.

Likewise:

Wall Closure Activity Workflow

covers the overall wall-close event.

Before Wall Closure

is the final STOP/CHECK gate.

So they complement one another.

4. Pre-Concealment view structure

Every concealment workflow should use exactly the same layout:

ABOUT TO BE CONCEALED

What will disappear?

VERIFY

Critical concealed-condition checks.

INTERFACES

Other systems that must be complete.

EVIDENCE

What must be captured now.

BLOCK CLOSURE IF

Conditions that should trigger review/control.

GATE

Applicable Build 3 gate.

AFTER RELEASE

What work may logically proceed next.

That's all.

5. No Quick / Full / Learn inside this screen

Pre-Concealment is already a specialized Quick workflow.

Do not add:

Quick | Full | Learn

again.

Instead, individual checklist items can link to their underlying activity pages.

Example:

Reinforcement →

opens:

2.2 Foundation Reinforcement — Quick

Back returns to:

Before Concrete Pour

Much cleaner.

6. PC-01 — BEFORE CONCRETE POUR

I would give concealment workflows their own IDs:

PC-*

So:

PC-CON-01 — Before Concrete Pour

ABOUT TO BE CONCEALED

- reinforcement;

- embeds;

- anchors;

- sleeves;

- conduits;

- grounding components;

- openings/blockouts;

- waterstops where applicable.

VERIFY

☐ Formwork geometry / condition acceptable
☐ Reinforcement complete and stable
☐ Cover / spacing / laps verified as required
☐ Embedded items complete
☐ Sleeves / openings correct
☐ Electrical conduits/cast-ins complete where applicable
☐ Grounding/bonding cast-ins complete where applicable
☐ Mechanical cast-ins complete where applicable
☐ Base/form clean and ready
☐ No unresolved clash or blocking issue

INTERFACES

Structural
Mechanical
Electrical
Survey
Testing

EVIDENCE

☐ Pour/element location
☐ Rebar condition
☐ Critical embeds / sleeves
☐ MEP cast-ins
☐ Pre-pour photos
☐ Required inspection/release evidence

BLOCK POUR IF

- missing/moved embed;

- unresolved reinforcement issue;

- unknown opening/sleeve;

- incomplete concealed MEP;

- unresolved technical change;

- required inspection/release incomplete;

- testing arrangement not ready.

GATE

G-STR-01 — Structural Pre-Pour Gate

Build 2 specifically identifies formwork, reinforcement, embedded items, site readiness, testing preparation and quality release as pre-pour controls.

AFTER RELEASE

→ Concrete Placement

7. PC-INT-01 — BEFORE WALL CLOSURE

ABOUT TO BE CONCEALED

- framing;

- backing;

- reinforcement;

- insulation;

- electrical rough-ins;

- mechanical rough-ins;

- sleeves/boxes;

- firestop;

- acoustic/fire assembly components;

- access provisions.

VERIFY

☐ Correct wall type / framing
☐ Openings correct
☐ Required backing installed
☐ Mechanical rough-in complete
☐ Electrical rough-in complete
☐ Services supported
☐ Required tests complete
☐ Insulation complete
☐ Penetrations complete
☐ Firestop complete where rated
☐ Access requirements resolved
☐ No unresolved coordination clash

INTERFACES

Architectural ↔ Mechanical
Architectural ↔ Electrical
Fire/Life Safety
Access & Maintainability

EVIDENCE

☐ Wall/location ID
☐ Rough-in photos
☐ Backing where important
☐ Firestop condition
☐ Concealed insulation
☐ Test/inspection evidence where required

BLOCK CLOSURE IF

- MEP incomplete;

- required test pending/failed;

- missing backing;

- unresolved penetration;

- incomplete firestop;

- access will be lost;

- rated assembly incomplete;

- blocking deficiency remains open.

GATES

G-MEP-01

G-INT-01

Build 2 explicitly requires final review of framing/masonry, openings, backing, MEP rough-ins, tests, fire integrity, insulation, access and unresolved issues before wall/ceiling closure.

AFTER RELEASE

→ Gypsum Board / Wall Closure

8. PC-INT-02 — BEFORE CEILING CLOSURE

ABOUT TO BE CONCEALED

- ductwork;

- piping;

- valves;

- conduits;

- cable trays;

- supports;

- fire/smoke dampers;

- firestop;

- insulation;

- controls;

- junction boxes;

- access requirements.

VERIFY

☐ Above-ceiling MEP complete
☐ Supports/hangers complete
☐ Required mechanical testing complete
☐ Concealed electrical work reviewed
☐ Fire/smoke dampers installed where applicable
☐ Damper access preserved
☐ Valves/access items remain accessible
☐ Firestop complete
☐ Insulation complete
☐ No unresolved clashes
☐ Ceiling devices coordinated

INTERFACES

HVAC
Plumbing
Electrical
Fire Alarm
Fire Protection
Ceiling
Access

EVIDENCE

☐ Area/room ID
☐ Above-ceiling photos
☐ Firestop
☐ Dampers
☐ Required tests
☐ Access condition

BLOCK CLOSURE IF

- required test incomplete;

- damper inaccessible;

- valve/control inaccessible;

- unfinished penetration;

- firestop incomplete;

- unsupported service;

- major clash remains;

- open blocking issue.

GATES

G-MEP-01

G-INT-01

AFTER RELEASE

→ Ceiling Grid/Panels Closure

9. PC-ROOF-01 — BEFORE COVERING A ROOF LAYER

This one is special because roof concealment happens repeatedly.

The user first chooses:

WHICH LAYER ARE YOU ABOUT TO COVER?

Deck/Substrate

Air/Vapour Layer

Insulation

Roof Membrane / Interfaces

Then the relevant gate opens.

A. Before Air/Vapour Layer

Derived from:

G-ROOF-01A

Verify:

☐ Deck/substrate accepted
☐ Openings coordinated
☐ Curbs coordinated
☐ Drains coordinated
☐ Surface clean/dry
☐ Damage corrected

Evidence:

☐ Overall substrate
☐ Openings/curbs/drains

Then:

→ 5.2 Air/Vapour Layer

B. Before Insulation

Derived from:

G-ROOF-01B

Verify:

☐ Control layer continuous
☐ Seams/laps complete
☐ Penetrations sealed
☐ Parapet/wall transitions complete
☐ Repairs complete

Evidence:

☐ Critical transitions
☐ Penetrations
☐ Repairs

Then:

→ Roof Insulation

C. Before Roofing Membrane

Derived from:

G-ROOF-01C

Verify:

☐ Correct insulation
☐ Thickness/layers
☐ Taper/slope
☐ Fastening
☐ No wet/damaged boards
☐ Drain sump geometry acceptable

Then:

→ Roofing Membrane

D. Before Equipment / Traffic Obscures Final Roof

Derived from:

G-ROOF-01D

Verify:

☐ Membrane complete
☐ Seams/laps accepted
☐ Flashings complete
☐ Penetrations complete
☐ Curbs complete
☐ Drains complete
☐ Repairs complete

Evidence:

☐ Membrane overall
☐ Penetrations
☐ Curbs
☐ Drains
☐ Flashings

Build 2 explicitly uses these successive roof inspection windows.

10. PC-EXT-01 — BEFORE BACKFILL

ABOUT TO BE CONCEALED

- underground service;

- joints;

- bedding;

- connections;

- structures;

- waterproofing/insulation where adjacent to foundation;

- service crossings;

- location/elevation information.

VERIFY

☐ Correct service/material
☐ Route/location correct
☐ Elevation correct
☐ Slope correct where applicable
☐ Bedding/support acceptable
☐ Joints/connections accepted
☐ Manholes/catch basins/connections complete
☐ Required tests passed
☐ Service protected
☐ Foundation waterproofing protected where applicable
☐ As-built/survey information captured

INTERFACES

Civil
Mechanical
Electrical
Foundation Waterproofing
Drainage

EVIDENCE

☐ Service ID
☐ Route/location
☐ Elevation/survey
☐ Connections/joints
☐ Test result
☐ Before-backfill photos

BLOCK BACKFILL IF

- test failed/pending;

- unaccepted connection;

- incorrect elevation/slope;

- damage;

- missing survey/as-built evidence;

- waterproofing/insulation damaged;

- blocking deficiency open.

GATE

G-EXT-01

Build 2 requires underground work to be inspected, tested, recorded and released before burial.

AFTER RELEASE

→ Backfill → Compaction

11. PC-FIRE-01 — BEFORE CLOSING A FIRE-RATED ASSEMBLY

This should be particularly rigorous.

ABOUT TO BE CONCEALED

- rated wall/floor/ceiling components;

- reinforcement/framing;

- penetrations;

- firestop;

- dampers;

- joint/head-of-wall conditions;

- concealed MEP.

VERIFY

☐ Correct rated assembly identified
☐ Required components/layers present
☐ Framing/masonry condition acceptable
☐ Penetrations identified
☐ Firestop system acceptable
☐ Opening/service configuration acceptable
☐ Firestop complete
☐ Damper installed where applicable
☐ Damper access maintained
☐ Rated door/frame interface coordinated where applicable
☐ No unprotected opening

EVIDENCE

☐ Assembly/location ID
☐ Penetration IDs/locations
☐ Firestop before concealment
☐ Approved system reference where required
☐ Dampers/interfaces
☐ Inspection evidence

BLOCK CLOSURE IF

- unidentified penetration;

- incomplete firestop;

- condition outside approved system;

- rated assembly incomplete;

- inaccessible damper;

- unresolved life-safety deficiency.

GATES

G-INT-01

Potentially contributes to:

G-LS-01

Build 2 treats firestop and rated assemblies as high-control pre-concealment work and explicitly requires reinspection when a controlled penetration is later modified.

AFTER RELEASE

→ Rated Assembly Closure

12. PC-MEP-01 — BEFORE CONCEALING MEP

This is the generic MEP shortcut.

Use when the user doesn't care whether the architectural closure is specifically a wall or ceiling.

ABOUT TO BE CONCEALED

- piping;

- ducts;

- conduits;

- cable trays;

- supports;

- joints;

- valves;

- dampers;

- insulation;

- controls;

- penetrations.

VERIFY

☐ Correct route / location
☐ Correct system/material
☐ Supports complete
☐ Joints/connections complete
☐ Required slope/alignment where applicable
☐ Penetrations complete
☐ Required tests complete
☐ Insulation complete where required
☐ Identification possible/complete where needed
☐ Access maintained
☐ No unresolved issue

EVIDENCE

☐ Location / system ID
☐ Concealed routing
☐ Joints/connections where important
☐ Required test
☐ Penetrations
☐ Photos before concealment

BLOCK CONCEALMENT IF

- failed/pending required test;

- unsupported system;

- incomplete joint;

- unresolved penetration;

- inaccessible maintainable component;

- unclosed issue.

GATE

G-MEP-01

AFTER RELEASE

→ Architectural closure / insulation / enclosure as applicable

13. Pre-Concealment mode should use gates as its primary source

This is crucial.

We do not manually maintain these checklists separately forever.

Architecture:

PreConcealmentWorkflow

↓

calls relevant Gate object

↓

extracts concealment-critical Activity content

↓

adds evidence priorities

↓

renders Pre-Concealment view

The gate remains authoritative for readiness logic.

14. Pre-Concealment object

Conceptually:

PreConcealmentWorkflow {

id

title

concealmentType

primaryGateIds[]

concealedActivityIds[]

conditionalActivityIds[]

verifyPriorities[]

evidencePriorities[]

blockingConditionRefs[]

downstreamActivityIds[]

}

Again:

no duplicated technical inspection content if the underlying activity already owns it.

15. Concealment workflow IDs

Freeze:

| ID | Workflow |
| --- | --- |
| PC-CON-01 | Before Concrete Pour |
| PC-INT-01 | Before Wall Closure |
| PC-INT-02 | Before Ceiling Closure |
| PC-ROOF-01 | Before Roof Layer |
| PC-EXT-01 | Before Backfill |
| PC-FIRE-01 | Before Closing Fire-Rated Assembly |
| PC-MEP-01 | Before Concealing MEP |

Seven is enough for MVP.

16. Evidence gets unusually high priority here

This should be one of the defining differences of Pre-Concealment Mode.

Normally:

inspect → evidence.

Here:

inspect + preserve what is about to disappear.

Evidence should therefore always ask:

LOCATION

Can someone identify exactly where this was?

OVERALL CONDITION

Can someone understand context?

CRITICAL DETAIL

Can someone see the hidden quality feature?

MEASUREMENT / SYSTEM ID

Where important.

TEST / RELEASE

Where required.

This turns photos from random site photography into quality evidence.

17. Photo guidance

For Pre-Concealment Mode, we can give a tiny universal photo reminder:

📷 CAPTURE

1. Overall

Shows context/location.

2. Detail

Shows critical item.

3. Interface

Shows relationship with adjacent work.

Not every condition needs exactly three photos, but this is a useful mental model.

18. No photo-storage system in MVP

Important boundary.

The app tells you:

what evidence is worth capturing.

It does not need to become:

camera + database + project record storage.

That would again create a parallel QMS.

19. Blocking issue semantics

Pre-Concealment Mode should use stronger language than normal Quick.

Not:

“Watch for missing firestop.”

Instead:

⛔ DO NOT CONCEAL / PROCEED UNTIL REVIEWED

if a hard blocking condition exists.

But with the universal-guide caveat:

Follow governing project procedure and authorized release requirements.

This does not give our guide contractual authority.

It tells the inspector the QC logic.

20. Three outcomes

At the bottom of any Pre-Concealment view:

🟢 READY FOR NEXT STEP

Applicable checks satisfied according to governing requirements.

🟡 REVIEW REQUIRED

Uncertain/conditional item or external acceptance remains.

🔴 NOT READY

Blocking condition identified.

Again, these are decision guidance states, not official project records.

21. Specialist/external acceptance

If a concealment condition involves specialist acceptance:

Example electrical/fire:

🔵 EXTERNAL / SPECIALIST ACCEPTANCE

Confirm applicable authorized inspection/release before concealment.

That preserves the authority boundary.

22. Conditional items

Very important.

Wall Closure does not always mean:

firestop required.

So:

☐ Firestop where rated penetration exists

Likewise:

☐ Acoustic insulation where required

☐ Dampers where applicable

☐ Vapour barrier where assembly requires

This keeps the workflow universal.

23. Hidden condition priority

When selecting Quick content for Pre-Concealment, use this ranking:

1.

Life safety / structural.

2.

Permanent embed / buried condition.

3.

Required test before cover.

4.

Critical interface.

5.

Access / maintainability.

6.

Evidence that cannot be recreated.

7.

General workmanship.

This is different from ordinary Quick extraction.

24. “Last responsible moment”

I would include one tiny contextual line:

LAST EASY CHECK

Example:

This is the last practical inspection point before concrete permanently conceals the embedded work.

or:

After ceiling closure, access to these services may become limited.

That reinforces why the shortcut matters.

No long lecture.

25. Invalidation warning

For selected workflows:

⚠ AFTER RELEASE

If later work modifies the concealed/accepted condition:

Re-review the affected assembly before relying on the previous acceptance.

Especially:

- firestop;

- envelope;

- roof;

- wall;

- test-dependent MEP.

This connects Step 6 naturally.

26. Pre-Concealment → underlying pages

Each major verification item should be tappable if deeper inspection criteria are needed.

Example:

Before Wall Closure

☐ Mechanical Rough-In →

Tap:

relevant Mechanical page — Quick

☐ Firestop →

Tap:

10.3 Firestopping — Quick

☐ Access →

Tap:

11.10 Access & Maintainability

This means the pre-concealment screen stays short while the full system remains one tap away.

27. Pre-Concealment should be location-agnostic

No:

Room 102
Wall W4

because the universal guide is not tracking project instances.

The screen is:

Before Wall Closure

and you mentally/apply it to whichever wall you're standing at.

Later we could optionally allow a temporary scratch label, but not MVP.

28. Pre-Concealment and Activity Mode overlap rule

When a composite Activity Mode workflow contains a concealment gate, show:

⚠ BEFORE PROCEEDING

Open Pre-Concealment Check →

Example:

Concrete Pour

→ Before Concrete Pour

Wall Closure

→ Before Wall Closure

This avoids duplicating the full final-gate checklist in Activity Mode.

29. Home-screen placement

I would rank homepage access approximately:

🔎 SEARCH

⚡ QUICK INSPECTION

🛠 WHAT ARE YOU DOING?

⚠ BEFORE CLOSING / COVERING

📚 BROWSE SYSTEMS

⭐ FAVORITES

Pre-Concealment deserves homepage-level access.

30. Search integration

Search:

before pour

→ PC-CON-01

close wall

→ PC-INT-01

backfill

→ PC-EXT-01

above ceiling

→ PC-INT-02

hide piping

→ PC-MEP-01

Later Build 6 search aliases make this very fast.

31. Pre-Concealment mode should work offline

Absolutely mandatory.

The entire point is:

you're in the field, at the exact moment something is about to disappear.

No connectivity dependency.

32. Pre-Concealment validation test

Before accepting a concealment workflow:

1.

Is this genuinely a concealment/irreversibility point?

2.

Did we identify what physically disappears?

3.

Are structural/life-safety items preserved?

4.

Are MEP interfaces preserved?

5.

Are required tests before concealment preserved?

6.

Are access requirements preserved?

7.

Is evidence emphasized?

8.

Are blocking conditions obvious?

9.

Is the applicable gate linked?

10.

Is the downstream activity obvious?

11.

Are conditional items properly qualified?

12.

Can the whole screen be understood in approximately 20–30 seconds?

If not, simplify.

33. What Pre-Concealment Mode does NOT do

Explicitly exclude:

- actual inspection sign-off;

- actual hold-point release;

- storing project photographs;

- assigning trades;

- generating project NCRs;

- storing official quality records;

- replacing PIE/PRIE;

- declaring contractual acceptance.

Again:

decision-support / field-reference tool.

34. Relationship with Quick

This distinction should remain intuitive:

Quick Mode

Inspect this activity.

Activity Mode

Coordinate this field event.

Pre-Concealment Mode

Stop before this disappears.

That is an excellent separation of concerns.

BUILD 5F — FROZEN ✓

We now have:

Pre-Concealment purpose ✓
Dedicated homepage shortcut ✓
Seven canonical concealment workflows ✓
PC- namespace ✓*
Concrete pre-pour ✓
Wall closure ✓
Ceiling closure ✓
Roof-layer progression ✓
Underground/backfill ✓
Fire-rated assembly closure ✓
Generic MEP concealment ✓
Gate-driven architecture ✓
Evidence-first behavior ✓
Photo mental model ✓
Blocking-condition logic ✓
Three decision states ✓
Specialist boundary ✓
Conditionality ✓
Last-responsible-moment cue ✓
Invalidation reminder ✓
Deep-link behavior ✓
Activity Mode overlap rule ✓
Homepage/search integration direction ✓
12-point validation test ✓
No parallel-QMS behavior ✓

The next step is 5G — Watch For / Don't Miss / QC Think. That one should be fairly compact: we need to freeze exactly how these three intelligence features are generated, how many items they can contain, how they differ, and how QC Think should behave consistently across every activity without becoming repetitive.

Yes. 5G is the field-intelligence layer — the part that keeps Quick Mode from becoming a dumb checklist.

We need to freeze three distinct tools:

WATCH FOR = what your eyes should hunt for.
DON’T MISS = what your brain absolutely must remember.
QC THINK = how to reason when the situation is not obvious.

They must stay clearly different.

BUILD 5 — STEP 5G

WATCH FOR / DON'T MISS / QC THINK

1. WATCH FOR — PURPOSE

Watch For is visual and observational.

It answers:

What physical defects or warning signs should I actively look for?

Typical sources:

- Build 2 common deficiencies;

- inspection criteria;

- practical failure examples;

- invalidation risks;

- recurring interface problems.

Examples:

Concrete

- displaced rebar;

- insufficient cover;

- missing embeds;

- segregation;

- poor consolidation.

Air Barrier

- tears;

- open seams;

- incomplete transitions;

- unsealed penetrations;

- later-trade damage.

Roofing

- wet substrate;

- punctures;

- bad laps/seams;

- incomplete flashing;

- ponding.

2. WATCH FOR — EXTRACTION RULES

An item qualifies if it is:

- directly observable;

- reasonably common;

- consequential;

- easy to miss;

- tied to a known Build 2 deficiency or failure mode.

Do not include abstract statements such as:

“non-compliance with specifications”

because that does not help the inspector look at the work.

Prefer:

“missing backing”

“blocked access”

“damaged coating”

“unsupported conduit”

3. WATCH FOR — ITEM LIMIT

Target:

3–6 items

Maximum:

8 for genuinely complex/high-control activities

If more than 8 are important, the activity probably needs:

- grouping;

- another activity page;

- Full Mode.

4. WATCH FOR — PRESENTATION

Quick Mode:

⚠ WATCH FOR

Missing backing · Oversized opening · Incomplete firestop · Later-trade damage

On tablet/phone, this could be small chips or compact bullets.

No explanations.

Tap an item later could jump to Learn's Common Failures explanation.

5. DON'T MISS — PURPOSE

Don't Miss is not visual-risk spotting.

It answers:

What 1–3 things would be especially costly, dangerous, or embarrassing to forget?

Typical triggers:

Concealment

Something is about to disappear.

Gate

Work should not progress without a key prerequisite.

Authority

Specialist acceptance is required.

Interface

One overlooked interface can undermine several systems.

Evidence

Evidence cannot easily be recreated afterward.

Invalidation

Later work can undo prior acceptance.

6. DON'T MISS — PRIORITY ORDER

When there are multiple candidates:

- Life safety / specialist authority

- Structural / irreversible concealment

- Major performance interface

- Required test / release

- Critical evidence / traceability

- Invalidation risk

Then keep only:

1–3 items

Usually 1 or 2.

7. DON'T MISS — EXAMPLES

Concrete

Verify every embedded item and MEP cast-in before pour release.

Firestop

Match the actual penetration condition to an approved system before concealment.

Electrical

Physical QC does not replace required authorized electrical acceptance.

Roof

Inspect penetrations and membrane condition before later equipment or traffic obscures them.

Underground

Capture location/elevation and required test evidence before backfill.

That is the right level of force.

8. WATCH FOR VS DON'T MISS

Freeze the distinction:

| Feature | Question |
| --- | --- |
| Watch For | What defects should my eyes hunt for? |
| Don't Miss | What critical point must I not forget before accepting/releasing? |

Example Firestop:

WATCH FOR

- oversized opening

- wrong system

- missing backing

- insufficient depth

- later damage

DON'T MISS

Confirm the installed condition matches the approved firestop system before concealment.

Perfect separation.

9. QC THINK — PURPOSE

QC Think is different from both.

It answers:

I’m not sure what to do here. What questions should guide my reasoning?

It should train professional judgment rather than encourage checklist dependency.

The six canonical questions should remain the same everywhere.

10. THE SIX QC THINK QUESTIONS

1. WHAT GOVERNS THIS?

Ask:

- drawing?

- specification?

- shop drawing?

- PIE/PRIE?

- approved submittal?

- manufacturer requirement?

- technical change?

This prevents inspection by memory alone.

2. WHAT IS ABOUT TO BECOME HIDDEN OR HARD TO ACCESS?

Ask:

- will concrete cover it?

- will drywall hide it?

- will ceiling closure block it?

- will backfill bury it?

- will insulation/cladding cover it?

- will equipment installation obscure it?

This directly reinforces the pre-concealment logic.

3. WHAT OTHER SYSTEM OR TRADE CAN THIS AFFECT?

Ask:

- structure?

- envelope?

- mechanical?

- electrical?

- fire/life safety?

- architectural finish?

- access/maintainability?

This forces multidisciplinary thinking.

4. WHAT EVIDENCE WOULD PROVE THIS WAS ACCEPTABLE?

Ask:

- photo?

- measurement?

- test result?

- inspection form?

- material ID?

- specialist acceptance?

- location/equipment ID?

This develops traceability thinking.

5. WHAT HAPPENS NEXT IF I ACCEPT THIS?

Ask:

- concrete pour?

- wall closure?

- insulation?

- equipment startup?

- commissioning?

- turnover?

This prevents premature acceptance.

6. HAS ANYTHING CHANGED SINCE THIS WAS LAST INSPECTED OR TESTED?

Ask:

- new penetration?

- repair?

- relocation?

- damage?

- revised drawing?

- material substitution?

- altered tested configuration?

This connects directly to Step 6 invalidation logic.

11. QC THINK SHOULD BE UNIVERSAL

The questions stay fixed.

What changes is the contextual hint.

Example:

Firestop

What governs this?
→ Approved firestop system / rated assembly requirements.

What becomes hidden?
→ Penetration configuration before wall closure.

What other system is affected?
→ Wall, mechanical/electrical service, life safety.

The question is universal; the hints are activity-specific.

12. QC THINK LEVELS

I recommend two layers.

LEVEL 1 — QUESTIONS ONLY

Default.

Shows the six questions.

LEVEL 2 — SHOW GUIDANCE

Tap one question to see context-specific prompts.

This avoids clutter while still making QC Think useful.

13. QC THINK SHOULD NOT GIVE FINAL AUTHORITY

Very important.

For specialist-sensitive work, guidance can say:

Check applicable project requirements and coordinate with authorized specialist.

It should not say:

“Therefore accept the electrical test.”

So QC Think supports reasoning, not unauthorized technical judgment.

14. QC THINK IN QUICK MODE

Quick should show only:

QC THINK ›

Tap → overlay/panel.

Not all six questions expanded permanently.

That keeps Quick lean.

15. QC THINK IN FULL MODE

Full can show a slightly richer version:

QC Think

- governing requirements

- concealment

- interfaces

- evidence

- downstream consequence

- changes since prior acceptance

Still collapsed by default.

16. QC THINK IN LEARN MODE

Learn can use it actively:

Apply QC Think to this example

Then show:

- situation;

- six questions;

- reveal reasoning.

That turns the framework into a professional-learning tool.

17. SOURCE DERIVATION

Watch For comes from:

- common deficiencies;

- inspection failures;

- practical examples;

- invalidation rules.

Don't Miss comes from:

- high-control items;

- gates;

- concealment;

- specialist boundaries;

- critical evidence;

- invalidation.

QC Think comes from:

- Build 3 architecture;

- Build 2 requirements/inspection/evidence logic;

- Step 6 invalidation.

No independent random content.

18. DATA MODEL

Conceptually:

FieldIntelligence {

watchFor[]

dontMiss[]

qcThink {

enabled

contextualHints {

governs

concealment

interfaces

evidence

downstream

changedSinceAcceptance

}

}

}

Simple.

19. WATCH FOR ITEM MODEL

Each item can internally store:

WatchForItem {

text

source

severity

relatedLearnTopic

}

No need to show metadata to the user.

20. DON'T MISS ITEM MODEL

DontMissItem {

text

priority

triggerType

relatedGateId?

specialistAlert?

}

Possible trigger types:

- concealment

- authority

- gate

- interface

- evidence

- invalidation

21. DEDUPLICATION RULE

If Watch For says:

missing firestop

Don't Miss should not merely repeat:

don't miss firestop.

It should elevate the principle:

Confirm the penetration is restored using the applicable approved system before concealment.

So:

Watch For = symptom
Don't Miss = governing critical principle

22. LANGUAGE STYLE

Watch For

Short noun phrases.

Good:

- wet insulation

- missing anchor

- blocked access

- open seam

Don't Miss

One concise imperative/reminder.

Good:

Capture evidence before concealment.

QC Think

Question form.

Good:

What happens next if I accept this?

This makes each feature visually distinct.

23. HIGH-CONTROL ACTIVITIES

For highControl nodes:

Watch For can expand slightly.

Don't Miss still stays 1–3.

QC Think may show a label:

High-control activity — consider Full Inspection

But no extra 15 warnings.

24. PRE-CONCEALMENT ACTIVITIES

If preConcealment = true, Don't Miss should strongly consider:

evidence before concealment

or:

gate/release before concealment

depending on the activity.

This is an extraction bias, not automatic wording.

25. SPECIALIST-INTERFACE ACTIVITIES

If specialistInterface = true, at least one of these should exist where material:

- Don't Miss specialist acceptance;

- QC Think specialist hint;

- Full specialist boundary card.

We should never lose the authority boundary through compression.

26. INVALIDATION-SENSITIVE ACTIVITIES

For selected activities:

Don't Miss

Reinspect if modified later.

or:

QC Think

Has anything changed since previous inspection/testing?

Examples:

- firestop;

- roofing membrane;

- tested piping;

- grounding;

- equipment startup.

27. WATCH FOR / DON'T MISS EXAMPLE — REINFORCEMENT

WATCH FOR

- displaced bars

- insufficient cover

- incorrect spacing

- poor lap condition

- missing chairs/support

- missing/misplaced embeds

DON'T MISS

Recheck reinforcement and embeds if anything is moved after the pre-pour inspection.

QC THINK

- What drawing/rebar detail governs?

- What will concrete permanently conceal?

- Are MEP embeds interacting with reinforcement?

- What photos/measurements prove condition?

- Is the pour gate truly ready?

- Has anything shifted since inspection?

28. EXAMPLE — AIR BARRIER

WATCH FOR

- tears

- open seams

- poor corner transition

- unsealed penetration

- incomplete opening transition

- later-trade damage

DON'T MISS

Check continuity at transitions and penetrations, not only the flat membrane.

QC THINK

- What detail governs this transition?

- What will cladding/insulation hide?

- Which penetration/opening trade affects continuity?

- What photos prove the interface?

- What layer comes next?

- Has another trade damaged previously accepted membrane?

29. EXAMPLE — HVAC EQUIPMENT

WATCH FOR

- wrong orientation

- poor anchorage

- missing vibration isolation

- blocked service clearance

- misaligned connection

- shipping/installation damage

DON'T MISS

Confirm practical service/removal access before surrounding work makes correction difficult.

QC THINK

- What approved equipment/submittal governs?

- What connections/access will become difficult later?

- Mechanical/electrical/controls interfaces?

- What evidence proves equipment ID and installation?

- Is startup the next step?

- Has equipment been moved since layout/base approval?

30. EXAMPLE — FAILED TEST

WATCH FOR

- wrong test boundary

- uncalibrated instrument

- incomplete isolation

- undocumented reading

- repair without retest

DON'T MISS

A correction does not close a failed test; preserve the failure-to-retest-to-acceptance chain.

QC THINK

- What procedure/acceptance criterion governs?

- What tested components will become inaccessible afterward?

- Which trade/system owns the failure?

- What records prove the full test history?

- What downstream work is blocked?

- Has the tested configuration changed?

31. USER INTERFACE BEHAVIOR

Quick:

⚠ Watch For (5)
❗ Don't Miss
🧠 QC Think

Watch For can be expanded inline.

Don't Miss stays visible.

QC Think opens a side panel/modal/accordion.

That hierarchy makes sense.

32. WATCH FOR SHOULD NOT BECOME ALARM FATIGUE

If every activity has:

⚠⚠⚠⚠⚠

the visual warning loses value.

So Watch For should use a modest warning icon, not giant red banners.

Reserve stronger red/blocking visuals for actual gate/Not Ready logic.

33. DON'T MISS SHOULD BE PROMINENT BUT SMALL

One small high-contrast callout.

Not a huge card.

Example:

❗ Don't Miss: Photograph the concealed firestop condition before wall closure.

Enough.

34. QC THINK SHOULD FEEL OPTIONAL

It is a reasoning aid, not another required step.

Do not force the six questions every time.

The user opens it when:

- uncertain;

- unfamiliar;

- dealing with an interface;

- reviewing a complicated issue;

- learning.

35. VALIDATION — WATCH FOR

Before approving a Watch For list:

- Is each item physically recognizable?

- Is each item actually tied to the activity?

- Is it consequential?

- Is wording concrete?

- Is it ≤6 normally?

- Did we avoid generic “non-compliance” language?

36. VALIDATION — DON'T MISS

- Is this genuinely critical?

- Would forgetting it materially affect quality?

- Is it different from Watch For?

- Is it 1–3 items?

- Does it preserve authority/concealment/gate logic where needed?

- Is wording concise?

37. VALIDATION — QC THINK

- Are all six questions available?

- Are hints activity-specific?

- Does it support reasoning rather than give fake certainty?

- Are specialist boundaries preserved?

- Does it surface concealment/interfaces/evidence/downstream/invalidation?

- Is it optional and unobtrusive?

38. FINAL ROLE OF THE THREE FEATURES

This is the simplest way to think about them:

WATCH FOR

Eyes

DON'T MISS

Memory

QC THINK

Judgment

That is exactly why all three belong.

BUILD 5G — FROZEN ✓

We now have:

Watch For purpose ✓
Visual-risk extraction ✓
Item limits ✓
Don't Miss purpose ✓
Priority hierarchy ✓
Watch For vs Don't Miss distinction ✓
Six fixed QC Think questions ✓
Contextual-hint behavior ✓
Quick/Full/Learn integration ✓
Specialist boundary preservation ✓
Pre-concealment bias ✓
Invalidation awareness ✓
Data model ✓
Deduplication rules ✓
Language style ✓
Validation rules ✓

The next step is 5H — one complete prototype activity. We should now pick one activity complex enough to stress-test the whole presentation system and build its Quick, Full, Learn, links, gates, Watch For, Don't Miss, QC Think, and pre-concealment behavior end to end. 10.3 Firestopping is probably the best test case because it touches architectural, mechanical, electrical, life safety, concealment, evidence, specialist boundaries, and invalidation all at once.

Exactly. 5H is where we prove that everything we designed from Build 3 through 5G actually works as one coherent field experience.

And 10.3 Firestopping is the right prototype because it stresses nearly every part of the architecture:

- multidisciplinary interfaces;

- pre-concealment;

- high-control inspection;

- evidence/traceability;

- conditionality;

- corrective action;

- reinspection;

- invalidation;

- wall/ceiling closure;

- specialist/approved-system boundary.

Build 2 itself says this should be one of the most rigorous modules in the entire application and makes the central point that firestop is not merely “red caulking around a pipe,” but an approved system involving the assembly, opening, penetrating item, annular space, backing/fill, product, depth/configuration and installation conditions.

BUILD 5 — STEP 5H

COMPLETE PROTOTYPE

10.3 FIRESTOPPING

A. CANONICAL ACTIVITY HEADER

10.3 FIRESTOPPING

High-Control · Pre-Concealment · Interface-Critical · Traceability-Critical

QUICK | FULL | LEARN

Quality Objective

Maintain the required performance of the rated assembly by ensuring each penetration corresponds to the applicable approved firestop system and is correctly installed, inspected, documented and maintained through later modifications.

Navigation

Before · Gate · Interfaces · Testing · After · Closeout

B. QUICK MODE

This is what you would normally open while standing beside the penetration.

FIRESTOPPING — QUICK

BEFORE

☐ Rated wall/floor assembly identified
☐ Required rating identified
☐ Penetrating item identified
☐ Approved/listed firestop system available
☐ Opening/configuration within applicable system limits
☐ Correct approved products available

Build 2 requires every penetration to establish the rated assembly, rating, penetrating item, opening and penetrant dimensions, annular space, approved system, backing/fill, firestop product, depth/thickness, installation sides and identification requirements.

INSPECT

☐ Assembly matches approved system
☐ Penetrating item/material/dimensions correct
☐ Opening size acceptable
☐ Annular space acceptable
☐ Backing type / placement / depth correct
☐ Exact approved firestop product
☐ Required installed depth / thickness
☐ Required configuration complete
☐ Finish continuous — no voids / incomplete perimeter
☐ Identification complete where required

These are directly distilled from Build 2's inspection sequence.

EVIDENCE

☐ Penetration ID / exact location
☐ Approved-system reference
☐ Critical dimensions where required
☐ Pre-concealment photographs
☐ Completed installation / label evidence

Build 2 specifically calls for contextual, opening/penetrant, backing/configuration, completed-firestop and identification-label photographs for critical penetrations, turning them into objective quality records rather than random photos.

⚠ WATCH FOR

- wrong product

- no applicable approved system

- excessive annular space

- wrong/missing backing

- insufficient installed depth

- missing collar/wrap

- incomplete perimeter

- added cables/services after acceptance

Those are all explicit Build 2 deficiency conditions.

❗ DON'T MISS

The actual penetration configuration must match the applicable approved system before concealment.

If another trade modifies the penetration later, previous acceptance cannot simply be assumed to remain valid.

Build 2 explicitly states that a previously accepted firestop can become noncompliant after modification.

GATE / NEXT

Pre-Concealment

→ G-INT-01 — Wall / Ceiling Pre-Concealment Gate

Where relevant, firestop completion is part of the condition permitting the rated wall/floor/ceiling assembly to be closed.

Next

→ Rated assembly closure / gypsum / ceiling closure as applicable.

RELATED (collapsed)

Interfaces

- 10.1 Fire-Rated Walls

- 10.2 Fire-Rated Floors / Ceilings

- 8.9 Mechanical Penetrations

- 9.9 Electrical Penetrations

- 11.7 Universal Penetrations

Related Workflow

- PC-FIRE-01 — Before Closing Fire-Rated Assembly

- WF-FIRE-01 — Firestop Inspection

QC THINK

What governs this?
Approved/listed firestop system, rated assembly requirements, firestop specification, approved submittal, PIE/PRIE.

What becomes hidden?
Backing, installed depth/configuration and sometimes the entire penetration after wall/ceiling closure.

What other system does this affect?
Rated wall/floor, mechanical/electrical service, architectural closure and life-safety performance.

What evidence proves compliance?
Penetration ID, system reference, dimensions where required, photos, inspection and identification.

What happens next if I accept it?
The rated assembly may be closed.

Has anything changed since previous inspection?
Added cable, changed service, enlarged opening, damaged firestop or other later-trade work should trigger review.

C. FULL MODE

10.3 FIRESTOPPING — FULL

At the top:

⚠ Pre-Concealment · High-Control · Recheck if Modified

Before | Gate | Interfaces | After | Closeout

QUALITY OBJECTIVE ▼

Ensure each firestopped penetration is controlled as an approved system—not merely by appearance—using the correct assembly, opening, penetrant, annular space, backing/fill, approved product, depth/configuration and installation conditions.

REQUIREMENTS ▶

Obtain

- firestop specification;

- approved firestop submittals;

- listed/approved systems;

- penetration schedule;

- rated wall/floor details;

- manufacturer instructions;

- approved engineering judgments where applicable;

- PIE/PRIE;

- installer requirements;

- inspection requirements.

For each penetration establish

- rated assembly;

- required rating;

- penetrating item;

- opening dimensions;

- penetrating-item dimensions;

- annular space;

- approved system;

- backing/fill;

- firestop product;

- required depth/thickness;

- installation side/configuration;

- identification requirements.

PLANNING ▶

Inspection planning

- identify penetrations requiring inspection;

- coordinate inspection before shafts/walls/ceilings become inaccessible;

- use unique penetration IDs where project procedures permit;

- establish status and system reference;

- plan photographic traceability.

Build 2 specifically proposes a Firestop Penetration Register and calls for inspection before areas become inaccessible.

Suggested register concept

ID | Location | Assembly | Rating | Service | Opening | Approved System | Status

We retain this concept in the guide, without pretending to replace the project's actual firestop register.

DOCUMENT CONTROL ▶

For each firestop system:

☐ Approved/listed system reference
☐ Current revision
☐ Correct rated assembly
☐ Correct penetrating item
☐ Dimensional limits
☐ Required products
☐ Required installation configuration

These are directly prescribed in Build 2.

MATERIAL / EQUIPMENT CONTROL ▶

Verify:

- manufacturer;

- exact product;

- batch/lot where required;

- shelf life/expiry where applicable;

- storage;

- backing material;

- mineral wool/fill;

- collars;

- wraps;

- sealants;

- identification labels.

A key Build 2 instruction:

Do not accept a product merely because its colour resembles other firestop.

INSPECTION / TESTING ▼

1. Rated Assembly

Verify the actual wall/floor assembly is the one covered by the applicable system.

2. Penetrating Item

Identify whether the penetration contains:

- pipe;

- conduit;

- cable;

- cable bundle;

- tray;

- duct;

- insulated pipe;

and verify the relevant material/dimensions.

3. Opening

Verify opening dimensions where required.

4. Annular Space

Verify against the dimensional limits of the applicable approved system.

5. Backing / Fill

Verify:

- type;

- density where specified;

- depth;

- placement.

6. Firestop Material

Confirm exact approved product.

7. Installed Depth / Thickness

Verify the required installed depth.

8. Configuration

Confirm the actual approved configuration, which may include as applicable:

- one side;

- both sides;

- collar;

- wrap;

- sealant;

- mineral wool;

- combination.

9. Finish

Verify:

- continuity;

- absence of voids;

- adhesion;

- complete perimeter.

10. Identification

Label/identify where required.

RECORDS / EVIDENCE ▶

Photographic Traceability

For critical penetrations, Build 2 proposes:

Photo 1 — Location / context

Photo 2 — Opening / penetrating item

Photo 3 — Backing / configuration before concealment

Photo 4 — Completed firestop

Photo 5 — Identification label where applicable

Retain

- penetration ID;

- location;

- rated assembly;

- penetrating item;

- approved system;

- dimensions where required;

- product;

- inspection;

- photographs;

- deficiency;

- repair;

- reinspection;

- closure.

NC / DEFICIENCY ▶

Common deficiencies

- wrong product;

- no approved system;

- excessive annular space;

- wrong backing;

- insufficient depth;

- missing collar/wrap;

- incomplete perimeter;

- incorrect penetrating-item configuration;

- damaged firestop;

- added cables after completion;

- penetration concealed before inspection.

Escalate when

- actual condition does not fit the available approved system;

- technical disposition/engineering judgment may be required;

- rated assembly is damaged or questionable;

- repeated modifications occur;

- concealed condition cannot be adequately verified;

- specialist/project approval requirements exceed normal field QC authority.

Open Deficiency / NC Workflow →

CORRECTIVE ACTION ▶

Build 2 gives a specific modification scenario:

An electrical contractor adds cables through an already completed penetration.

The QC workflow is:

Identify firestop ID → reopen quality status → compare modified configuration against approved system → remove/repair as required → reinstall approved configuration → reinspect → photograph → update register → re-close.

The key rule is:

Never assume the original acceptance remains valid.

VERIFICATION ▶

After correction, confirm:

- actual configuration matches approved system;

- repair complete;

- no voids;

- dimensions acceptable;

- identification updated;

- photographs complete.

CLOSURE ▶

The penetration is ready for closure only when applicable items are confirmed:

☐ Penetration ID
☐ Assembly identified
☐ Rating identified
☐ Approved system confirmed
☐ Penetrating item correct
☐ Opening acceptable
☐ Annular space acceptable
☐ Backing correct
☐ Product correct
☐ Depth/thickness correct
☐ Configuration correct
☐ Identification complete
☐ Photographs complete
☐ Deficiencies closed

COMMUNICATIONS ▶

This portion is structurally derived from the Build 2 workflow and our Step 7 communications schema.

Before

- coordinate inspection before closure;

- confirm relevant trade(s);

- confirm required witness/project inspection process.

During

- clarify system/configuration questions;

- coordinate rated-assembly and MEP interface issues.

Issue / Escalation

- responsible trade;

- quality responsible person;

- applicable specialist/professional where approved system/disposition is uncertain.

After

- communicate accepted/rejected/rework status;

- communicate whether closure may proceed under governing project process.

OUTPUTS ▶

At completion, the QC process should leave:

Records

- identifiable penetration record;

- applicable system reference;

- inspection evidence;

- photographs.

Acceptance Evidence

- completed inspection/reinspection;

- deficiencies closed;

- identification/documentation complete.

Follow-Up where necessary

- repair;

- reinspection;

- technical disposition;

- updated penetration record.

REPORTING / ANALYSIS ▶

Build 2 proposes tracking:

- total penetrations;

- inspected;

- accepted;

- rejected;

- awaiting repair;

- awaiting specialist disposition;

- modified after acceptance;

- recurring defect types;

- recurring responsible trade.

This is an excellent managerial-quality transition:

repeated firestop problems may reveal coordination, sequencing, installer or process-control issues rather than isolated defects.

QUALITY CHECKPOINT ▶

Build 2's final checkpoint is:

☐ System reference
☐ Assembly
☐ Rating
☐ Penetrating item
☐ Opening
☐ Annular space
☐ Backing
☐ Product
☐ Depth
☐ Configuration
☐ Finish
☐ Label
☐ Photos
☐ Reinspection
☐ Closure

That remains the Full Mode final self-audit.

D. LEARN MODE

FIRESTOPPING — LEARN

WHAT IS THIS?

Firestopping is the controlled restoration of a rated assembly where a service penetrates it.

The important mental model is:

Firestop is a system, not simply a visible sealant product.

The accepted condition depends on the combination of the rated assembly, penetrating item, opening, annular space, backing/fill, specific firestop products, depth and installation configuration.

WHY IT MATTERS

A penetration changes a rated wall or floor.

Therefore quality control has to answer:

- Is the correct rated assembly identified?

- What passes through it?

- Does the actual geometry fit the applicable approved system?

- Was the required system actually installed?

- Can we prove the concealed condition?

- Has anyone changed it afterward?

Build 2 explicitly summarizes the firestop logic in almost exactly this sequence.

KNOW THE TERMS

Rated Assembly

The wall/floor/ceiling construction whose required fire-resistance condition is being maintained.

Penetrating Item

The pipe, conduit, cable, tray, duct or other service passing through the assembly.

Annular Space

The space between the penetrating item and the edge of the opening.

Backing / Fill

Material installed in the penetration as part of the approved system configuration.

Installed Depth / Thickness

Required depth of the firestop material within the assembly/system.

Approved / Listed System

The defined configuration against which the actual penetration is compared.

Collar / Wrap

System components used in applicable penetration configurations.

These terms come directly from the components Build 2 requires the inspector to establish and inspect.

HOW GOOD WORK LOOKS

A well-controlled firestop penetration should have:

- identifiable location;

- known rated assembly/rating;

- correctly identified service;

- opening geometry compatible with the approved system;

- correct backing/fill;

- exact approved product/components;

- correct installed depth/configuration;

- complete perimeter without voids;

- required identification;

- photographic/inspection traceability;

- no unresolved deficiency before concealment.

CRITICAL CHECKS EXPLAINED

1. Approved System

The product alone is not the acceptance basis.

The entire configuration matters.

A firestop product of the same colour or manufacturer does not automatically make an installation acceptable. Build 2 specifically warns against accepting firestop based merely on appearance/colour.

2. Opening & Annular Space

An approved firestop system has dimensional limits.

Therefore an installation can appear fully sealed while still having geometry that does not correspond to the approved configuration.

Build 2 explicitly instructs the inspector to verify annular space against approved-system limits.

3. Backing

Backing is not simply invisible filler.

The applicable system may depend on:

- type;

- density;

- depth;

- placement.

Because backing often becomes hidden beneath the visible firestop surface, this is a strong pre-concealment inspection/evidence point.

4. Installed Depth

A finished surface can look complete without proving the required depth exists underneath.

That is why depth/thickness must be verified where required before the configuration becomes inaccessible.

5. Later Modification

Firestop acceptance is not permanent if another trade changes the penetration.

Adding cables or otherwise modifying the opening/service configuration changes the condition that was previously inspected. Build 2 explicitly requires the quality status to be reopened, the modified configuration checked against the approved system, repaired as needed, reinspected and re-documented.

COMMON FAILURES

Wrong Product

What you see:
Product does not match approved system documentation.

Why it matters:
Appearance alone does not establish system compliance.

Verify next:
System reference, manufacturer/product and configuration.

Excessive Annular Space

What you see:
Large gap around service.

Why it matters:
Actual opening geometry may fall outside approved system limits.

Verify next:
Opening dimensions, service dimensions and applicable system limits.

Wrong / Missing Backing

What you see:
Backing absent, poorly placed or inconsistent with required configuration.

Why it matters:
Backing forms part of the firestop system.

Verify next:
Type, density where specified, depth and placement.

Insufficient Installed Depth

What you see:
Surface appears filled but depth may be inadequate.

Why it matters:
Required material depth is part of system configuration.

Verify next:
Applicable system detail and field measurement.

Incomplete Perimeter / Voids

What you see:
Gaps or discontinuities remain.

Why it matters:
Installation is not complete around the penetration.

Verify next:
Finish continuity, voids, adhesion and complete perimeter.

Added Cable After Acceptance

What you see:
Existing firestop has been disturbed to pass another cable/service.

Why it matters:
The previously accepted configuration is no longer necessarily the inspected/approved condition.

Verify next:
Reopen the penetration quality status and evaluate the new configuration.

PRACTICAL EXAMPLE

Situation

Electrical contractor adds cables through an already completed firestop.

Observation

The penetration was previously accepted, but its service configuration has changed.

Quality Concern

The previous approved configuration may no longer represent the actual penetration.

Reasoning

Ask:

- Which penetration is this?

- Which rated assembly?

- Which system previously applied?

- Does the new cable configuration still fit that system?

- Was backing/firestop disturbed?

- Does repair or a different approved system/disposition apply?

Action Path

Identify ID → reopen quality status → compare modified configuration → repair/reinstall appropriate approved configuration → reinspect → photograph → update record → re-close.

That is directly the Build 2 example.

Lesson

Acceptance belongs to the inspected configuration, not permanently to the hole in the wall.

INTERFACES & SEQUENCE

Rated Wall / Floor

Provides the assembly whose performance must be restored.

Mechanical / Electrical

Creates the service penetration.

Firestop

Restores the applicable rated penetration condition.

Architectural Closure

Can make the penetration/backing/configuration inaccessible.

Life Safety

Firestop becomes part of the overall rated-compartmentation quality condition.

Universal Penetrations

Links firestop with structural, envelope, insulation and finish restoration where applicable.

Sequence:

Rated Assembly → Opening → Service → Firestop System → Inspection/Evidence → Closure

And if modified later:

Modification → Reopen → Reinspect/Repair → Evidence → Re-close

SPECIALIST / AUTHORITY BOUNDARY

QC Role

- verify current approved system information is available;

- compare physical condition to applicable system;

- verify products, geometry, backing, depth/configuration and finish;

- document/evidence condition;

- identify conditions outside clear approved configuration;

- control/escalate unresolved conditions.

Authorized Technical / Project Role

Where the actual condition falls outside approved/listed system limits or requires engineering judgment/disposition, use the authorized project process rather than improvising a repair.

Build 2 explicitly includes approved engineering judgments among possible governing firestop information.

WHAT CAN UNDO ACCEPTANCE?

- new cable/service;

- enlarged opening;

- removed firestop;

- damaged installation;

- altered backing/configuration.

Result:

Re-review the affected penetration rather than relying on the previous acceptance.

QUALITY MANAGEMENT INSIGHT

Build 2 recommends tracking accepted, rejected, repair-pending, specialist-disposition-pending and modified penetrations, as well as recurring defect types and recurring responsible trades.

So repeated firestop failures should eventually lead to questions such as:

Is there a coordination problem?

Are penetrations being created too late?

Is one trade repeatedly modifying accepted work?

Are installers using the wrong approved-system information?

That is the jump from inspection to quality management.

E. PRE-CONCEALMENT INTEGRATION

If the user enters:

⚠ BEFORE CLOSING FIRE-RATED ASSEMBLY

the firestop component should appear roughly as:

FIRESTOP

☐ Rated assembly/rating identified
☐ Every penetration identified
☐ Approved system confirmed
☐ Opening/annular space acceptable
☐ Backing/configuration complete
☐ Required depth complete
☐ Identification complete
☐ Pre-concealment photographs captured
☐ No unresolved firestop deficiencies

Open Firestopping →

This takes the user to:

10.3 Firestopping — Quick

So Pre-Concealment stays concise.

F. ACTIVITY MODE INTEGRATION

Under:

WF-FIRE-01 — Firestop Inspection

the mission view can show:

Prepare

- identify rated areas;

- approved-system information;

- penetration register/scope.

Inspect
→ 10.3 Firestopping

Interfaces

- Mechanical Penetrations

- Electrical Penetrations

- Rated Walls/Floors

- Universal Penetrations

Evidence

- ID/location

- system reference

- critical photos

Issue
→ Deficiency / NC

After
→ Wall/Ceiling Closure

Again, no duplicate firestop technical content.

G. BUILD 3 RELATIONSHIP BEHAVIOR

For 10.3, the prototype confirms that our graph architecture works.

Direct Interfaces

10.3 ↔ 10.1 Fire-Rated Walls

10.3 ↔ 10.2 Fire-Rated Floors/Ceilings

10.3 ↔ 8.9 Mechanical Penetrations

10.3 ↔ 9.9 Electrical Penetrations

10.3 ↔ 11.7 Universal Penetrations

Gate

10.3 → G-INT-01 where closure depends on completed firestop.

Downstream

Rated assembly closure.

Invalidation

Later penetration modification can reopen 10.3 and the affected rated assembly.

No contradictions emerge.

H. 5H PROTOTYPE VALIDATION

Now test the architecture against the criteria we created.

| Test | Result |
| --- | --- |
| Quick understandable in seconds? | PASS |
| Quick preserves critical checks? | PASS |
| Quick preserves evidence? | PASS |
| Watch For is visual? | PASS |
| Don't Miss is genuinely critical? | PASS |
| Full retains Build 2 detail? | PASS |
| Full avoids giant wall of text? | PASS |
| Learn explains rather than repeats? | PASS |
| Interfaces make sense? | PASS |
| Pre-concealment works? | PASS |
| Activity Mode integrates cleanly? | PASS |
| Gate behavior works? | PASS |
| Invalidation works? | PASS |
| Specialist boundary preserved? | PASS |
| No invented numerical criteria? | PASS |
| No project-specific hold point invented? | PASS |
| No duplicate deficiency workflow? | PASS |
| No duplicate technical content? | PASS |
| Build 2 remains source of technical content? | PASS |
| Universal-guide boundary preserved? | PASS |

So I would call this prototype successful.

I. ONE IMPORTANT DESIGN IMPROVEMENT DISCOVERED

5H reveals something useful:

Quick Mode should show a tiny critical flags row immediately below the title.

For this example:

⚠ Pre-Concealment · ◆ High-Control · ↔ Interface-Critical

This gives context before you read the checklist.

We don't need many possible flags.

I would limit the visible flag set to:

- Pre-Concealment

- High-Control

- Specialist

- Testing

- Interface-Critical

Everything else remains metadata.

This is a genuine improvement worth carrying forward from the prototype.

J. ANOTHER IMPROVEMENT: “RECHECK IF MODIFIED”

For invalidation-sensitive activities such as:

- firestop;

- roofing membrane;

- air barrier;

- tested piping;

- grounding;

- equipment after start-up;

I would add a standardized compact line:

↻ Recheck if modified

Tap → explanation.

That is much cleaner than squeezing invalidation prose into every Quick page.

So we gain one reusable UI device from 5H.

K. FINAL FIRESTOP PAGE ARCHITECTURE

What the user actually experiences is very simple:

Firestopping

⚠ Pre-Concealment · ◆ High-Control · ↔ Interface-Critical · ↻ Recheck if Modified

QUICK

Before → Inspect → Evidence → Watch For → Don't Miss → Gate/Next → Related → QC Think

FULL

Requirements → Planning → Documents → Materials → Inspection → Evidence → Issues → Corrective Action → Verification → Closure → Communications → Outputs → Analysis → Checkpoint

LEARN

What Is This → Why It Matters → Terms → Good Work → Critical Checks Explained → Common Failures → Example → Interfaces → Authority Boundary → What Can Undo Acceptance

That is exactly the system we were trying to create.

BUILD 5H — COMPLETE ✓

And importantly, the prototype didn't expose a structural flaw requiring us to go back and redesign Build 3 or 5A–5G. The architecture held together.

So Build 5 as a whole is now architecturally complete.